DESIGN REPORT – BOM ID: EP 1581

FLANGED ANGLE STOP VALVE WITH MANUAL CONTROL, Ti ALLOY NB150 NP40

M/S ULTRA DIMENSIONS PVT. LTD., VISAKHAPATNAM

| Prepared by: |  | Checked by: | Approved by: (DMDE) |
| --- | --- | --- | --- |
| Guravaiah V CAE Engr | Aniket N CAE Engr | (Ashok Varma R) (Sr. General Manager) |  |

Table of Contents

1.	Revision Log	10

2.	Scope	11

2.1.	Details of Type of Analysis	11

3.	Software/Tools	12

4.	References	13

5.	Equipment Details	14

5.1.	CAD Model	14

5.2.	Mass, Material, Centre of gravity and model orientation	19

5.2.1.	The weight in FE analysis is tabulated below	19

5.2.2.	Centre of gravity	20

5.2.3.	Model Orientation	20

5.3.	Details of Structural Connections to Surrounding Structure	21

5.4.	Working Principle	21

5.4.1.	Core working of the valve	21

5.4.2.	Load path of the valve for various loading conditions	21

5.5.	Components	22

5.5.1.	Body	22

5.5.2.	Disc	22

5.5.3.	Gland	22

5.5.4.	Bonnet	22

5.5.5.	Flanges	22

5.6.	Design Specifications	23

6.	Modelling Details	24

6.1.	Details of all the non-structural parts which are excluded from the model	24

6.2.	Modelling approach for all structural parts	24

6.2.1.	Simulation Geometry	24

6.2.2.	FE Mesh	25

6.2.3.	Mesh Quality criteria	27

6.3.	Modelling approach for bolted joints	29

6.4.	Modelling approach for welded joints	30

6.4.1.	3D Solid Modelling with Shared Topology and Conforming Mesh (Higher-Fidelity Weld Simulation)	30

6.5.	Modelling approach for joints	31

6.6.	Modelling approach for gaskets	39

6.7.	Modelling approach for interfaces (contacts)	39

6.8.	Details of materials models selected	39

7.	Details of the Loading and BC	40

7.1.	Load Case – 1 MBD Analysis	40

7.1.1.	MBD Analysis Results	40

7.2.	Load Case – 2 Modal Analysis	42

7.2.1.	Modal Analysis Methodology	42

7.2.2.	Modal Analysis Boundary Conditions	43

7.2.3.	Modal Analysis Results	43

7.2.4.	Modal Analysis Conclusions	49

7.3.	Load Case - 3 Static Structural Analysis	50

7.3.1.	Static Structural Analysis Boundary Conditions	50

7.3.2.	Static Structural Analysis Loading Conditions	53

7.3.3.	Static Structural Analysis Results	53

7.3.4.	Static Structural Analysis Conclusions	68

7.4.	Load case - 4 Vibration resistance analysis	68

7.4.1.	Vibration Resistance Analysis Methodology	69

7.4.2.	Vibration Resistance Analysis Boundary Conditions	69

7.4.3.	Vibration Resistance Analysis Loading Conditions	70

7.4.3.1.	Load Case - 3.1 Vibration Resistance Analysis in X- Direction	70

7.4.3.2.	Load Case - 3.2 Vibration Resistance Analysis in Y- Direction	72

7.4.3.3.	Load Case - 3.3 Vibration Resistance Analysis in Z- Direction	73

7.4.4.	Vibration Resistance analysis Results	75

7.4.4.1.	Analysis Harmonic In X direction	75

7.4.4.2.	Analysis Harmonic In Y direction	85

7.4.4.3.	Analysis Harmonic In Z direction	94

7.4.5.	Harmonic Analysis Conclusions	102

7.5.	Load Case - 5 Shock Analysis - Transient Approach	103

7.5.1.	Shock Analysis Boundary And Loading Conditions For Transient Analysis	103

7.5.2.	Shock Pulse Specifications	107

7.5.3.	Results of Shock Analysis	108

7.5.3.1.	Analysis Transient in X Direction	108

7.5.3.2.	Analysis Transient in Y Direction	122

7.5.3.3.	Analysis Transient in Z Direction	136

7.5.4.	Shock Analysis Conclusion	148

8.	CFD Analysis	150

8.1.	Introduction	150

8.2.	CFD Analysis Methodology	151

8.3.	Assumptions	151

8.4.	CFD Mesh	152

8.5.	Material Selection	153

8.6.	Turbulent Viscous model	153

8.7.	Boundary Conditions	154

8.8.	Solution Methods	155

8.8.1.	Convergence criteria	155

8.9.	Results	156

8.9.1.	Y+ Plot	156

8.9.2.	Velocity distribution	157

8.9.3.	Pressure distribution	158

8.9.4.	Calculation of valve coefficient	158

8.9.5.	CFD Analysis Conclusions	159

9.	Material Properties	160

10.	Design calculations of the Valve	161

10.1.	Pressure Rating of the Valve	161

10.2.	Design Validation Valve Wall Thickness Calculations	161

10.3.	Design Validation of the Union Bolted Joint and (Body- Bonnet) Flange	161

10.4.	Calculation of the Bolt strength and Preload	161

10.5.	Flange moments	161

10.6.	Effort Required	161

10.7.	End Connection Flange Design	161

10.8.	End Connection Flange Design	161

List of Figures

Figure 1 - Equipment Details: CAD Model	14

Figure 2 Equipment Details: CAD Model Section View	15

Figure 3 Fluid Volume	16

Figure 4 Item no as Per BOM	17

Figure 5 - Valve Component Description: (BOM)	18

Figure 6 - Model Orientation: Standard Earth Gravity	20

Figure 7 - Simulation Geometry: Valve Geometry for FE Analysis	24

Figure 8 - FE Mesh: Mesh for FE Simulation	26

Figure 9 Element Quality	27

Figure 10 Aspect Ratio	28

Figure 11 Jacobian Ratio	28

Figure 12 Skewness	29

Figure 13 Maximum Corner Angle	29

Figure 14 Share Topology at Pipe and Flange Weld Locations	30

Figure 15 - Modelling approach for joints: Revolute - 2 Disc To 7 Spindle	31

Figure 16 - Modelling approach for joints: Cylindrical - 15 Yoke Sleeve To 10 Stem	32

Figure 17 - Modelling approach for joints: Fixed - 10 Stem To 50 Cylindrical Pin-2	33

Figure 18 - Modelling approach for joints: Revolute - 14 Bonnet To 15 Yoke Sleeve	34

Figure 19 - Modelling approach for joints: Revolute - 14 Bonnet To 19 Gear	35

Figure 20 - Modelling approach for joints: Revolute - 14 Bonnet To 20 Pinion Shaft	36

Figure 21 - Modelling approach for joints: Revolute - 18 Yoke Nut To 15 Yoke Sleeve	37

Figure 22 - Modelling approach for joints: Revolute - 20 Pinion Shaft To 33 Indicator	38

Figure 23 - Modelling approach for joints: General - 12 Bush-1 To 10 Stem	39

Figure 24  Loads Applied in MBD analysis	40

Figure 25 Joint Force at Threaded Joint	41

Figure 26 Joint force at Yoke Sleeve Bearings	41

Figure 27 Joint Forces at Pinion Shaft Bushes	42

Figure 28 - Modal Analysis: Boundary Conditions Modal Analysis	43

Figure 29 - Modal Analysis: Mode 1 at Frequency 36.139 Hz	46

Figure 30 - Modal Analysis: Mode 2 at Frequency 145.71 Hz	47

Figure 31 - Modal Analysis: Mode 3 at Frequency 173.41 Hz	47

Figure 32 - Modal Analysis: Mode 4 at Frequency 188.83 Hz	48

Figure 33 - Modal Analysis: Mode 5 at Frequency 258.18 Hz	48

Figure 34 - Modal Analysis: Mode 6 at Frequency 267.52 Hz	49

Figure 35 - Static Structural Analysis: Boundary Conditions Static Structural Analysis	50

Figure 36 - Static Structural: Total Deformation - End Time	53

Figure 37 - Static Structural: Equivalent (von-Mises) Stress - End Time	54

Figure 38 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time	54

Figure 39 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time	55

Figure 40 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time	56

Figure 41 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time	57

Figure 42 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time	58

Figure 43 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time	59

Figure 44 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time	60

Figure 45 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time	61

Figure 46 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time	62

Figure 47 - Static Structural: All - Force Reaction - Fixed Support - End Time	63

Figure 48 - Static Structural: All - Moment Reaction - Fixed Support - End Time	64

Figure 49 - Harmonic Response X: Boundary Conditions	70

Figure 50 - Harmonic Response X: Displacement Amplitude (X)	70

Figure 51 - Harmonic Response Y: Boundary Conditions	72

Figure 52 - Harmonic Response Y: Displacement Amplitude (Y)	72

Figure 53 - Harmonic Response Z: Boundary Conditions	73

Figure 54 - Harmonic Response Z: Displacement Amplitude (Z)	74

Figure 55 - Harmonic Response X: Equivalent (von-Mises) Stress	75

Figure 56 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5	76

Figure 57 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5	76

Figure 58 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66	77

Figure 59 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS970_EN19	77

Figure 60 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS2874_CA104	78

Figure 61 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS970_EN36C	78

Figure 62 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100	79

Figure 63 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15	79

Figure 64 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100	80

Figure 65 - Harmonic Response X: Frequency Response_MAT_ASTM_B367_Gr_5	80

Figure 66 - Harmonic Response X: Frequency Response_MAT_ASTM_B348_Gr_5	81

Figure 67 - Harmonic Response X: Frequency Response_MAT_ASTM_D4066_Nylon66	81

Figure 68 - Harmonic Response X: Frequency Response_MAT_BS970_EN19	82

Figure 69 - Harmonic Response X: Frequency Response_MAT_BS2874_CA104	82

Figure 70 - Harmonic Response X: Frequency Response_MAT_BS970_EN36C	83

Figure 71 - Harmonic Response X: Frequency Response_MAT_ASTM_A276_S32100	83

Figure 72 - Harmonic Response X: Frequency Response_MAT_ASTM_A217_CA15	84

Figure 73 - Harmonic Response X: Frequency Response_MAT_ASTM_A240_S32100	84

Figure 74 - Harmonic Response Y: Equivalent (von-Mises) Stress	85

Figure 75 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5	85

Figure 76 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5	86

Figure 77 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66	86

Figure 78 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS970_EN19	87

Figure 79 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS2874_CA104	87

Figure 80 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS970_EN36C	88

Figure 81 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100	88

Figure 82 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15	89

Figure 83 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100	89

Figure 84 - Harmonic Response Y: Frequency Response_MAT_ASTM_B367_Gr_5	90

Figure 85 - Harmonic Response Y: Frequency Response_MAT_ASTM_B348_Gr_5	90

Figure 86 - Harmonic Response Y: Frequency Response_MAT_ASTM_D4066_Nylon66	91

Figure 87 - Harmonic Response Y: Frequency Response_MAT_BS970_EN19	91

Figure 88 - Harmonic Response Y: Frequency Response_MAT_BS2874_CA104	92

Figure 89 - Harmonic Response Y: Frequency Response_MAT_BS970_EN36C	92

Figure 90 - Harmonic Response Y: Frequency Response_MAT_ASTM_A276_S32100	93

Figure 91 - Harmonic Response Y: Frequency Response_MAT_ASTM_A217_CA15	93

Figure 92 - Harmonic Response Y: Frequency Response_MAT_ASTM_A240_S32100	94

Figure 93 - Harmonic Response Z: Equivalent (von-Mises) Stress	94

Figure 94 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS970_EN19	95

Figure 95 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS2874_CA104	95

Figure 96 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS970_EN36C	96

Figure 97 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100	96

Figure 98 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15	97

Figure 99 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100	97

Figure 100 - Harmonic Response Z: Frequency Response_MAT_ASTM_B367_Gr_5	98

Figure 101 - Harmonic Response Z: Frequency Response_MAT_ASTM_B348_Gr_5	98

Figure 102 - Harmonic Response Z: Frequency Response_MAT_ASTM_D4066_Nylon66	99

Figure 103 - Harmonic Response Z: Frequency Response_MAT_BS970_EN19	99

Figure 104 - Harmonic Response Z: Frequency Response_MAT_BS2874_CA104	100

Figure 105 - Harmonic Response Z: Frequency Response_MAT_BS970_EN36C	100

Figure 106 - Harmonic Response Z: Frequency Response_MAT_ASTM_A276_S32100	101

Figure 107 - Harmonic Response Z: Frequency Response_MAT_ASTM_A217_CA15	101

Figure 108 - Harmonic Response Z: Frequency Response_MAT_ASTM_A240_S32100	102

Figure 109 - Transient Analysis: Boundary Conditions	103

Figure 110 - Transient: Transient - Shock Load	104

Figure 111 - Transient Analysis: Boundary Conditions	105

Figure 112 - Transient: Transient - Shock Load	105

Figure 113 - Transient Analysis: Boundary Conditions	106

Figure 114 - Transient: Transient - Shock Load	106

Figure 115 - Shock Analysis: Shock Pulse	107

Figure 116 - Transient X: Total Deformation - End Time	108

Figure 117 - Transient X: Equivalent (von-Mises) Stress - End Time	109

Figure 118 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time	110

Figure 119 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time	111

Figure 120 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time	112

Figure 121 - Transient X: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time	113

Figure 122 - Transient X: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time	114

Figure 123 - Transient X: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time	115

Figure 124 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time	116

Figure 125 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time	117

Figure 126 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time	118

Figure 127 - Transient Y: Total Deformation - End Time	122

Figure 128 - Transient Y: Equivalent (von-Mises) Stress - End Time	123

Figure 129 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time	124

Figure 130 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time	125

Figure 131 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time	126

Figure 132 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time	127

Figure 133 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time	128

Figure 134 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time	129

Figure 135 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time	130

Figure 136 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time	131

Figure 137 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time	132

Figure 138 - Transient Z: Total Deformation - End Time	136

Figure 139 - Transient Z: Equivalent (von-Mises) Stress - End Time	137

Figure 140 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time	137

Figure 141 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time	138

Figure 142 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time	139

Figure 143 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time	140

Figure 144 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time	141

Figure 145 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time	142

Figure 146 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time	143

Figure 147 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time	144

Figure 148 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time	145

Figure 149 (a) Schematic of valve and its (b) sectional view.	150

Figure 150 Extracted fluid domain of EP 1581Valve.	151

Figure 151 CFD Mesh Size	152

Figure 152 Polyhedral grids generated on the exterior surface and hexahedral grids on the internal surface of the valve.	153

Figure 153 Material properties of sea-water.	153

Figure 154 Selection of turbulence model.	154

Figure 155 Mass Balance.	155

Figure 156 Residual plots.	156

Figure 157 Y+ contours on the computational domain.	156

Figure 158 Velocity contours in the computational region of EP 1581.	157

Figure 159 Velocity streamlines in the computational region of EP 1581.	158

Figure 160 Pressure contours in the computational region of EP 1581.	158

Figure 161 Area averaged pressure obtained at the inlet and outlet	158

List of Tables

Table 1 - Revision Log	10

Table 2 - Scope of Design Analysis	11

Table 3 - Software/Tools Used	12

Table 4 - Standard References	13

Table 5 - Weight Balance Table	19

Table 6 - Design Specifications	23

Table 7 - Mesh Parameters	25

Table 8 - Mesh Quality Criteria	27

Table 9 MBD Analysis Boundary Conditions	40

Table 10 Joint Force Summary Table	42

Table 11 – Modal Analysis Boundary Conditions	43

Table 12 - Ratio of Effective Mass to Total Mass	45

Table 13 - Modal Frequencies	46

Table 14 - Modal Analysis Summary	49

Table 15 - Static Structural Boundary Conditions	51

Table 16 - Boundary Conditions Summary - Static Structural	51

Table 17 - All - Force Reaction - Fixed Support - End Time - Tabular Data	63

Table 18 - All - Moment Reaction - Fixed Support - End Time - Tabular Data	64

Table 19 - Bolt Force Summary Table	64

Table 20 - Static Structural Results Summary	68

Table 21 - Vibration Proof Characteristics	69

Table 22 - Displacement Amplitude (X)	71

Table 23 - Vibration Resistance Analysis Settings (X)	71

Table 24 - Displacement Amplitude (Y)	72

Table 25 - Vibration Resistance Analysis Settings (Y)	73

Table 26 - Displacement Amplitude (Z)	74

Table 27 - Vibration Resistance Analysis Settings (Z)	74

Table 28 - Static Structural Results Summary	102

Table 29 - Analysis Settings	104

Table 30 - Analysis Settings	105

Table 31 - Analysis Settings	107

Table 32 - Shock Pulse Specification Table	107

Table 33 - Bolt Loads: Shock in X direction	118

Table 34 - Bolt Loads: Shock in Y direction	132

Table 35 - Bolt Loads: Shock in Z direction	145

Table 36 - Transient Shock Response Summary	148

Table 37 Boundary conditions.	155

Table 38 Parameters used for calculation of valve coefficient.	159

# Revision Log

Table 1 - Revision Log

| Document Revision number | Date of revision | Description | Remarks |
| --- | --- | --- | --- |
| R00 | 29-Apr-26 | Initial Submission |  |

# Scope

EQUIPMENT NAME: FLANGED ANGLE STOP VALVE WITH MANUAL CONTROL, Ti ALLOY NB150 NP40

BOM ID: EP 1581

PO No with date: HSV/33/Ti-VVS (28 TYPES-PROTO)/S5/360 dated 28 Jan 26

## Details of Type of Analysis

Table 2 - Scope of Design Analysis

| S. No. | Analysis Type | Section (In this Document) | Relevant Section of PO | Remarks |
| --- | --- | --- | --- | --- |
| Scope of the FE Analysis to check for induced stress at test/strength pressure |  |  |  |  |
| 1 | Modal Analysis | Section 7.1 | Section - B |  |
| 2 | Static Analysis | Section 7.2 | Section - B |  |
| 3 | Harmonic Analysis (Vibration Resistance) | Section 7.3 | Section - B |  |
| 4 | Shock Analysis | Section 7.4 | Section - B |  |
| 5 | Multi Body Dynamics | Section 7.5 | Section-B |  |
| 6 | ABN/SBN | NA | Section-B | Will Be Performed Experimentally |
| 7 | Estimation of the Co-efficient of Hydraulic Resistance through CFD | Section 8 | Section-B |  |
| Hand Calculations |  |  |  |  |
| 7 | Valve Pressure Rating | Section 10 | Section - B | NA |
| 8 | Valve Wall Thickness |  | Section - B |  |
| 9 | Design of Union Joint |  | Section - B |  |
| 10 | Bolt Pretensions |  | Section - B |  |
| 11 | Effort Calculations |  | Section - B |  |
| 12 | End Flange Selection |  | Section - B |  |

# Software/Tools

Table 3 - Software/Tools Used

| S. No. | Software | System configuration | Versions |
| --- | --- | --- | --- |
| 1 | SolidWorks | Dual Intel Xeon E5-2680; 128GB RAM; NVIDIA Quadro P4000 | 2020 |
| 2 | Ansys | Dell Precision 7820; Xeon Gold 6138; 20 cores; 128GB RAM | 2024 R1 |

# References

Table 4 - Standard References

| S. No. | Reference type (Standard Journal paper etc.) | Reference Number | Title | Revision year | Referred for |
| --- | --- | --- | --- | --- | --- |
| 1 | Standard | Standard ASME PCC 1 | Guidelines for Pressure Boundary Bolted Joint Assembly | 2019 | Bolt Pretentions |
| 2 | Standard | ASME section Sec II Part D | Boiler and Pressure Vessels 2023 | 2023 | Material Properties, and other design aspect |
| 3 | Standard | ASME Sec VIII Div 1 | Gasket Seating | 2023 | Gasket selection |
| 4 | Standard | ASME B16.34 | Valve body thickness | 2020 | Valve body thickness |
| 5 | Standard | ASME Sec VIII Div 2-part 5 | Stress Linearization | 2023 | Stress Linearization |
| 6 | Standard | IEC 60534-2-1/ ANSI/ISA-75.01.01 | Flow Capacity Sizing 2011 | 2011 | CV calculations |
| 7 | Standard | GOST 1536 76/ ASME Sec VIII Div1 | Specification for Valves for interface dimensions |  | Flange Design |
| 8 | BOOK |  | Design of Valve and Fittings by GH Pearson |  | Effort calculations |

# Equipment Details

## CAD Model

Figure 1 - Equipment Details: CAD Model

Figure 2 Equipment Details: CAD Model Section View

Figure 3 Fluid Volume

Figure 4 Item no as Per BOM

Figure 5 - Valve Component Description: (BOM)

## Mass, Material, Centre of gravity and model orientation

### The weight in FE analysis is tabulated below

Table 5 - Weight Balance Table

| Sl. No. | Body Name | Material | Mass (FE Weight) (kg) | Dwg. Weight (kg) |
| --- | --- | --- | --- | --- |
| 1 | 1 Body | ASTM B 367 Gr.5 | 46.87267 | 47.6 |
| 2 | 2 Disc | ASTM B 348 Gr.5 | 5.07028 | 5.15 |
| 3 | 3 Disc Seal | ASTM D4066- Nylon 66 | 0.01454 | 0.01 |
| 4 | 4 Piston | ASTM B 348 Gr.5 | 4.98349 | 5.11 |
| 5 | 5 Key | BS 970 EN19 | 0.02636 | 0.03 |
| 6 | 5 Key | BS 970 EN19 | 0.02636 | 0.03 |
| 7 | 7 Spindle | ASTM B 348 Gr.5 | 1.62764 | 1.65 |
| 8 | 8 Split Ring | ASTM B 348 Gr.5 | 0.07357 | 0.07 |
| 9 | 8 Split Ring | ASTM B 348 Gr.5 | 0.07357 | 0.07 |
| 10 | 10 Stem | ASTM B 348 Gr.5 | 1.25449 | 1.28 |
| 11 | 11 Stem Cover | ASTM B 348 Gr.5 | 9.40327 | 9.55 |
| 12 | 12 Bush-1 | BS 2874 CA 104 | 1.66385 | 1.68 |
| 13 | 14 Bonnet | ASTM B 367 Gr.5 | 18.72133 | 19.05 |
| 14 | 15 Yoke Sleeve | BS 2874 CA 104 | 3.0184 | 3.35 |
| 15 | 17 Lock Nut | ASTM B 348 Gr.5 | 0.86686 | 0.88 |
| 16 | 18 Yoke Nut | ASTM B 348 Gr.5 | 0.78264 | 0.80 |
| 17 | 19 Gear | BS 970 EN36C | 2.87994 | 2.95 |
| 18 | 20 Pinion Shaft | BS 970 EN36C | 1.15555 | 1.19 |
| 19 | 22 Ring Nut | ASTM A 240 S32100 | 0.16414 | 0.17 |
| 20 | 25 Top Cover | ASTM A 217 CA15 | 4.76128 | 4.88 |
| 21 | 26 Bush-3 | BS 2874 CA 104 | 0.11761 | 0.13 |
| 22 | 28 Hand Wheel | ASTM A 217 CA15 | 3.63539 | 3.37 |
| 23 | 30 Slotted Nut-1 | BS 2874 CA 104 | 0.02331 | 0.03 |
| 24 | 32 Side Cover | ASTM A 240 S32100 | 0.59525 | 0.59 |
| 25 | 33 Indicator | BS 2874 CA 104 | 0.04599 | 0.05 |
| 26 | 34 Slotted Nut-2 | BS 2874 CA 104 | 0.02217 | 0.03 |
| 27 | 45 Bush-2 | BS 2874 CA 104 | 0.03995 | 0.04 |
| 28 | 45 Bush-2 | BS 2874 CA 104 | 0.03995 | 0.04 |
| 29 | 50 Cylindrical Pin-2 | ASTM B 348 Gr.5 | 0.01388 | 0.01 |
| 30 | Outlet Pipe | ASTM B 367 Gr.5 | 11.10508 | - |
| 31 | Outlet Flange | ASTM B 367 Gr.5 | 4.83997 | - |
| 32 | Inlet Pipe | ASTM B 367 Gr.5 | 11.10508 | - |
| 33 | Inlet Flange | ASTM B 367 Gr.5 | 4.83997 | - |
| 34 | Gasket 1 | ASTM D4066- Nylon 66 | 0.01534 | - |
| 35 | Gasket 2 | ASTM D4066- Nylon 66 | 0.01534 | - |
|  | Total Weight From 1 To 29 |  | 107.9737 | 109.79 |

### Centre of gravity

a)	Centroid Location is (X, Y, Z) = (201.2666, 102.1355, 320.2737) mm

b)	Limiting Dimensions (L X B X H): (405 X 320 X 832) mm

### Model Orientation

Standard earth gravity of 9806.6 mm/s² is applied in the -Z Direction.

Note: As per the instructions and information provided by the equipment manufacturer, the valve is required to remain in the close position during operation. Therefore, only the physics corresponding to this common operating condition has been simulated in the present analysis.

Figure 6 - Model Orientation: Standard Earth Gravity

## Details of Structural Connections to Surrounding Structure

a) Valve assembly is connected to the piping by flanges at the both ends with 12 sets of M20 bolts.

b) Body of the valve and Bonnet is connected through 12 sets of M24 bolts with Nuts and Tab washers. Fluid flow happens through the body of the valve.

c) Stem cover is seated in the body of the valve. Stem cover allows the axial moment of Spindle assembly to the body of the valve. O-rings are there to arrest fluid leakage, between Stem cover and spindle.

d) Spindle assembly is fixed to the Stem at the top. The rotation of hand wheel is converted to axial motion of the spindle assembly through Stem via gear drive.

e) Stem is connected to the bonnet through the Yoke Sleeve. Yoke Sleeve is fixed to the bonnet at the outer side through bearings; the inner side of the Yoke Sleeve allows the motion of the stem through the threads. The Yoke Sleeve is rotated through the gear placed at top. The Gear is driven by the handwheel through pinion shaft.

f) Disc is fixed with the spindle at the bottom. Disc moves axially inside the valve body to regulate the flow. At the fully closed condition, the disc gets seated on the valve body.

## Working Principle

### Core working of the valve

A flanged angled stop valve operates by moving the disc in a linear motion perpendicular to the flow of the fluid to either block, allow, or regulate its passage. The term "angle-way" indicates that the fluid flows in a perpendicular path through the valve body, minimizing pressure loss when fully open.

### Load path of the valve for various loading conditions

Under normal operating conditions, the dominant loading consists of internal pressure, piping reactions, and self-weight. The pressure loading on the cavity produces membrane and bending stresses in the valve body shell.

These loads are subsequently transferred through the valve body to the flange regions / support interface, where they are reacted by the adjoining piping and structural supports. Piping axial forces, bending moments, and torsional effects applied at the flanges are directly transmitted into the body, resulting in localized stress intensification at the flange neck and body transitions.

For modal analysis, the objective is to characterize the global and local vibration response of the valve structure. Excitation is assumed to be transmitted primarily through the support/flange interface from the connected piping system or structural support. The valve body forms the primary structural path for vibration transmission, while the stem disc assembly behaves as a coupled internal subsystem.

In harmonic analysis, imposed base motion or force excitation applied at the support locations propagates through the body and bonnet assembly. Dynamic amplification is expected in regions of stiffness discontinuity, such as the bonnet connection, flange transitions, and stem interface. The disc and stem experience secondary vibration response due to their mechanical coupling with the bonnet and seat.

For transient shock or impulsive loading scenarios, the excitation is transmitted to the valve primarily through the support or flange boundary or through CG. The shock input induces rapid acceleration of the valve body, producing inertia forces throughout the mass of the structure. These inertia effects dominate the structural response rather than conventional static force transfer.

The body mass reacts first, followed by delayed inertial response of the bonnet, stem, and disc assembly due to relative motion and compliance within the structure. Stress concentrations are expected at regions of geometric discontinuity, including flange necks, bonnet attachment areas, and body seat transitions. The transient loads are ultimately transmitted back through the supports to the adjoining piping or structural framework.

## Components

### Body

Function: The body houses all internal components and has flanged ends for connecting to the pipeline. The straight design positions the inlet and outlet is at a 180-degree.

### Disc

Function: The disc is the movable component that allows or blocks fluid flow. It lifts to allow fluid flow in the desired direction and closes to prevent backflow.

### Gland

Function: Gland is to compress the gland packing, creating a seal around the spindle to prevent fluid leakage.

### Bonnet

Function: The bonnet covers the top of the valve body and houses the nut-stud assembly and other components. It can be removed for maintenance.

### Flanges

Function: The flanges are integral parts of the valve body, drilled according to standard flange patterns (such as ANSI, DIN, or JIS) for easy bolting to the pipeline. Gaskets are used between the flanges and the pipeline to ensure a leak-proof connection.

## Design Specifications

Table 6 - Design Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Diameter Nominal (DN) | 150 | mm |
| Pressure Nominal (PN) | 4 | MPa |
| End Connection | FLANGE to FLANGE |  |
| Type | ANGLE STOP VALVE |  |
| Flowrate | 110 | m3/h |
| Working Medium | Sea Water |  |
| Working Temperature | Negative 2 to Positive 50 | ℃ |
| Weight of Valve | 90.8 ± 10% | Kg |
| Material | TITANIUM ALLOY |  |
| Manufacturing Method | Casting |  |

# Modelling Details

## Details of all the non-structural parts which are excluded from the model

The non-structural and soft components such as O-ring, split pin, washers, Felt Seal, Nuts, Body Plugs will be excluded in the FE model, as their contribution to overall structural strength and stiffness is negligible. The exclusion of these parts simplified the model while maintaining sufficient accuracy for evaluating the valves structural integrity under operating and transient loading conditions.

## Modelling approach for all structural parts

The Valve components must be compatible with sea water environment. The valve must withstand operating loads and environmental loads i.e., shock loads and propeller induced vibrations.

This analysis aims to address the following key aspects:

Structural Integrity: Perform a structural analysis of the Valve to determine stresses, deformations, vibration responses, and potential failure modes. Ensure that the valve's components can withstand the mechanical forces generated during operation and environmental loads.

FE model has been made like the geometry. Only features like markings, logos and threads are de-featured in the model for geometry simplification. Connecting Flanges and pipes are simulated with the valve assembly.

### Simulation Geometry

The geometry considered for the FE analysis is shown below.

Figure 7 - Simulation Geometry: Valve Geometry for FE Analysis

### FE Mesh

Table 7 - Mesh Parameters

| Sl. No. | Sizing Name | Element Size |
| --- | --- | --- |
| 1 | 28 Hand Wheel | 9 |
| 2 | 30 Slotted Nut-1 | 3 |
| 3 | 20 Pinion Shaft | 5 |
| 4 | 25 Top Cover | 9 |
| 5 | 26 Bush-3 | 5 |
| 6 | 19 Gear | 2 |
| 7 | 22 Ring Nut | 3 |
| 8 | 15 Yoke Sleeve | 5 |
| 9 | Keys | 2 |
| 10 | 14 Bonnet | 7 |
| 11 | 18 Yoke Nut | 5 |
| 12 | 45 Bush-2 | 2 |
| 13 | 1 Body | 10 |
| 14 | 10 Stem | 5 |
| 15 | 12 Bush-1 | 5 |
| 16 | 11 Stem Cover | 7 |
| 17 | 7 Spindle | 5 |
| 18 | 2 Disc | 7 |
| 19 | 4 Piston | 5 |
| 20 | 17 Lock Nut | 5 |
| 21 | split ring | 3 |
| 22 | 50 Cylindrical Pin-2 | 1 |
| 23 | 3 Disc Seal | 2 |
| 24 | Flanges | 7 |
| 25 | Pipes | 5 |
| 26 | 32 Side Cover | 7 |
| 27 | 34 Slotted Nut-2 | 3 |
|  | Total Elements | 1138767 |
|  | Total Nodes | 1916079 |

Figure 8 - FE Mesh: Mesh for FE Simulation

The pipe is meshed with the 2 quadratic elements across the thickness with minimum 5 nodes to cater the bending mode of the pipe.

### Mesh Quality criteria

Table 8 - Mesh Quality Criteria

| Parameter | Ideal Range | Acceptable Range | Status |
| --- | --- | --- | --- |
| Element Quality | ~ 1 | >0.75 for 75% of Elements | Within the acceptance range |
| Aspect Ratio | 1 to 5 | <= 10 (Structural) | Within the acceptance range |
| Skewness | 0-0.25 | <= 0.85 (warning), <= 0.95 (error) | Within the acceptance range |
| Jacobian Ratio (Structural) | ~ 1 | <= 40 | Within the acceptance range |
| Determinant of Jacobian (FEA) | Positive | > 0 | Within the acceptance range |
| Taper (Structural) | ~ 0 | <= 0.5 | Within the acceptance range |
| Maximum Angle (Triangles/Quads) | 60-90 deg | <= 120 deg | Within the acceptance range |

Figure 9 Element Quality

Figure 10 Aspect Ratio

Figure 11 Jacobian Ratio

Figure 12 Skewness

Figure 13 Maximum Corner Angle

## Modelling approach for bolted joints

a) Bonnet to Body: Beam element is modelled between Bonnet and Body connection and meshed with BEAM188 elements.

b) Flange to Flange: Beam element is modelled between Flange-to-flange connection and meshed with BEAM188 elements.

Washers of 1.5D are be created around the bolt seating areas. Bolt length was reduced by scoping the tap hole (bolts end at half of the height of the Tap Hole). The beam element is tied to the nut contact area with the connecting body.

Note: The forces (Axial) as well as Shear) at the joint interfaces will be provided in the final design report.

## Modelling approach for welded joints

### 3D Solid Modelling with Shared Topology and Conforming Mesh (Higher-Fidelity Weld Simulation)

The second methodology applies a full 3D solid modelling approach, with shared topology across welded regions to generate a conforming mesh and common interface nodes. This provides a physically realistic bonded representation of welded continuity without reliance on contact definitions.

This method is particularly suitable for cases involving severe loading such as vibration, harmonic excitation, transient shock, seismic actions, or high operating pressure levels. It provides improved capability to evaluate localized stresses and structural response near welded regions, while still maintaining code consistent treatment of welds as continuous structural connections, unless a specific weld fatigue or fracture assessment is required.

Figure 14 Share Topology at Pipe and Flange Weld Locations

## Modelling approach for joints

The valve handle has 1 DOF, and it rotates around its axis. The revolute joint in the valves will be modelled using the connections, Insert body to body joint approach.  The stationary part such as valve bonnet will be considered as scope and moving part like handle to be mobile.

The local coordinate system will be used, where its Z axis will be aligned to the axis of the valve stem.

Figure 15 - Modelling approach for joints: Revolute - 2 Disc To 7 Spindle

Figure 16 - Modelling approach for joints: Cylindrical - 15 Yoke Sleeve To 10 Stem

Figure 17 - Modelling approach for joints: Fixed - 10 Stem To 50 Cylindrical Pin-2

Figure 18 - Modelling approach for joints: Revolute - 14 Bonnet To 15 Yoke Sleeve

Figure 19 - Modelling approach for joints: Revolute - 14 Bonnet To 19 Gear

Figure 20 - Modelling approach for joints: Revolute - 14 Bonnet To 20 Pinion Shaft

Figure 21 - Modelling approach for joints: Revolute - 18 Yoke Nut To 15 Yoke Sleeve

Figure 22 - Modelling approach for joints: Revolute - 20 Pinion Shaft To 33 Indicator

Figure 23 - Modelling approach for joints: General - 12 Bush-1 To 10 Stem

## Modelling approach for gaskets

Frictional contacts are used for metal to gasket contacts at counter flange and gasket interface, with friction coefficient of 0.15. Gaskets will be modelled with linear material properties using the actual thickness. The Friction coefficient for Nylon to SS pair is very low and lies in the range of 0.05 to 0.15.

## Modelling approach for interfaces (contacts)

All other parts without sliding motions were considered as bonded contact. The contacts between spindle and contacting bodies except in threaded regions where relative motion is required are modelled as no separation. Dry Friction coefficient for Metal-to-metal contacts is 0.5, For metal to gasket contacts 0.15 is taken as friction coefficient.

## Details of materials models selected

For all the materials, Linear Isotropic material models were considered as the analysis is performed for elastic limits. Working temperature is between -2 ℃ to 50 ℃, which is deemed as not high to affect material properties.

# Details of the Loading and BC

## Load Case – 1 MBD Analysis

The multibody dynamic analysis is performed to find the forces generated at the joints which can be used to apply on joints while solving the static structural analysis. In this the actual behaviour of the joints is modelled in order to facilitate the actual motion.

Table 9 MBD Analysis Boundary Conditions

| Sl. No. | Loading or BC | Location | Value |
| --- | --- | --- | --- |
| A | Standard Earth Gravity | CG | 9.8066 m/s2 |
| B | Rotation | Handwheel | 360 degrees |

Figure 24  Loads Applied in MBD analysis

### MBD Analysis Results

The Joint force at bushes, bearings and screw joints is evaluated and used as input in the Static Analysis. The other joint’s reaction is not required as these also generated in the static analysis, due to applied forces.

Figure 25 Joint Force at Threaded Joint

Figure 26 Joint force at Yoke Sleeve Bearings

Figure 27 Joint Forces at Pinion Shaft Bushes

Table 10 Joint Force Summary Table

| Sr. No. | Location | Force [N] |
| --- | --- | --- |
| 1 | Screw Joint at Yoke Sleeve and Stem | 25.211 N |
| 2 | Bearings at Yoke Sleeve | 231.51 N |
| 3 | Bushes at Pinion Shaft | 323.37 N |

## Load Case – 2 Modal Analysis

The modal analysis is performed to find the mode shapes and the fundamental natural frequencies of the structures, valves in this case. The objective is to obtain the mass participation factor of more than 80% for more mass to be included in succeeding the harmonic and transient analysis.

### Modal Analysis Methodology

The Finite Element (FE) modal analysis has been meticulously conducted, employing the Block Lanczos method, with a comprehensive selection of modes. Specifically, a substantial number of modes have been included in the analysis to ensure that the cumulative modal mass participation exceeds 80% across all three mutually perpendicular directions encompassing both translations and rotations.

a) Modal Analysis without any operating loads will be performed to find the actual frequency and mode shapes.

b) Ensure at least 80 % Mass participation is achieved. We may limit the mass participation below 80 %, if the resultant frequencies are deemed to be high enough.

### Modal Analysis Boundary Conditions

Table 11 – Modal Analysis Boundary Conditions

| Sl. No. | Loading or BC | Location | Value |
| --- | --- | --- | --- |
| A | Fluid Mass | Distributed at Wet Surface | 33.456 Kg |
| B | Fixed | Pipe End | - |
| C | SKF Bearings | 15 Yoke Sleeve |  |

Figure 28 - Modal Analysis: Boundary Conditions Modal Analysis

### Modal Analysis Results

The modal analysis results consist of two components the mass participation summary. The mass participation summary can be used to evaluate the number of modes to extract. The bottom line for mass participation factor is 80%. Here the mass participation of above 80% is achieved in non con strained directions. The Table Ratio of Effective Mass to Total Mass shows the achieved ratio of Effective Mass to Total Mass. This indicate that the number of extracted Modes are sufficient to Mode Superposition Analysis.

Table 12 - Ratio of Effective Mass to Total Mass

| Mode | Frequency [Hz] | X Direction | Y Direction | Z Direction | Rotation X | Rotation Y | Rotation Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 36.1387 | 0.325557E-05 | 0.370223E-05 | 0.767487 | 0.146251 | 0.249109 | 0.107570E-05 |
| 2 | 145.711 | 0.272464 | 0.939157E-02 | 0.773764E-05 | 0.500242E-02 | 0.142975 | 0.345692 |
| 3 | 173.408 | 0.263757E-04 | 0.391281E-04 | 0.931298E-01 | 0.187182 | 0.327936E-01 | 0.716412E-04 |
| 4 | 188.827 | 0.646335E-12 | 0.594445E-06 | 0.728887E-02 | 0.277976E-02 | 0.128337E-02 | 0.691161E-05 |
| 5 | 258.181 | 0.162929E-02 | 0.184519E-01 | 0.418334E-05 | 0.920896E-02 | 0.970181E-03 | 0.695307E-02 |
| 6 | 267.516 | 0.388191E-02 | 0.841938E-03 | 0.239594E-01 | 0.644488E-02 | 0.267657E-01 | 0.685120E-05 |
| 7 | 268.538 | 0.344161E-01 | 0.343381E-02 | 0.139935E-02 | 0.601686E-03 | 0.117888E-01 | 0.117000E-02 |
| 8 | 275.806 | 0.389463E-01 | 0.826500 | 0.774496E-03 | 0.421128 | 0.358216E-01 | 0.229722 |
| 9 | 278.178 | 0.749725E-04 | 0.312926E-01 | 0.175022E-01 | 0.388296E-02 | 0.513663E-01 | 0.103383E-01 |
| 10 | 322.200 | 0.563423 | 0.208977E-01 | 0.567214E-04 | 0.105227E-01 | 0.285919 | 0.909246E-01 |
| 11 | 485.968 | 0.343933E-07 | 0.407465E-07 | 0.121671E-04 | 0.231116E-04 | 0.870633E-04 | 0.505632E-07 |
| 12 | 538.006 | 0.824538E-03 | 0.565595E-02 | 0.879669E-07 | 0.303271E-02 | 0.359520E-03 | 0.530252E-01 |
| 13 | 598.077 | 0.237830E-05 | 0.114854E-04 | 0.507823E-02 | 0.295986E-01 | 0.971462E-02 | 0.456856E-06 |
| 14 | 668.519 | 0.731068E-07 | 0.826787E-08 | 0.618420E-06 | 0.589065E-05 | 0.170980E-04 | 0.178446E-07 |
| 15 | 668.590 | 0.210300E-05 | 0.587222E-06 | 0.422163E-07 | 0.332667E-06 | 0.159995E-05 | 0.290250E-07 |
| 16 | 668.922 | 0.795817E-08 | 0.302850E-08 | 0.205700E-05 | 0.324436E-06 | 0.112963E-05 | 0.229445E-08 |
| 17 | 669.021 | 0.266963E-06 | 0.300518E-06 | 0.235152E-07 | 0.113930E-06 | 0.255792E-06 | 0.820482E-07 |
| 18 | 688.418 | 0.724297E-02 | 0.461220E-02 | 0.288383E-05 | 0.225497E-02 | 0.412364E-02 | 0.105460E-02 |
| 19 | 718.434 | 0.862893E-04 | 0.173682E-03 | 0.945484E-05 | 0.790563E-04 | 0.118556E-03 | 0.166197E-01 |
| 20 | 746.961 | 0.107582E-08 | 0.399540E-05 | 0.207906E-01 | 0.161625E-03 | 0.267545E-01 | 0.138541E-05 |
| Sum |  | 0.923024 | 0.921311 | 0.937506 | 0.828160 | 0.879970 | 0.755588 |

Table 13 - Modal Frequencies

| Mode | Frequency [Hz] |
| --- | --- |
| 1. | 36.139 |
| 2. | 145.71 |
| 3. | 173.41 |
| 4. | 188.83 |
| 5. | 258.18 |
| 6. | 267.52 |
| 7. | 268.54 |
| 8. | 275.81 |
| 9. | 278.18 |
| 10. | 322.2 |
| 11. | 485.97 |
| 12. | 538.01 |
| 13. | 598.08 |
| 14. | 668.52 |
| 15. | 668.59 |
| 16. | 668.92 |
| 17. | 669.02 |
| 18. | 688.42 |
| 19. | 718.43 |
| 20. | 746.96 |

Figure 29 - Modal Analysis: Mode 1 at Frequency 36.139 Hz

Figure 30 - Modal Analysis: Mode 2 at Frequency 145.71 Hz

Figure 31 - Modal Analysis: Mode 3 at Frequency 173.41 Hz

Figure 32 - Modal Analysis: Mode 4 at Frequency 188.83 Hz

Figure 33 - Modal Analysis: Mode 5 at Frequency 258.18 Hz

Figure 34 - Modal Analysis: Mode 6 at Frequency 267.52 Hz

### Modal Analysis Conclusions

The Modal Analysis is performed to extract the modes of vibration of the structure. The Fundamental frequency of vibration was found to be 36.139Hz.  As this is less than 160 Hz therefore the Transient Shock approach will be used to perform the Shock Analysis.

Table 14 - Modal Analysis Summary

| Sl. No | Frequency (Hz) | Operating Frequency | Dominant in Direction | Remark |
| --- | --- | --- | --- | --- |
| 1. | 36.14 | 3 to 33 Hz | Rotation about X & Rotation about Y Direction & Z Direction | Not in or near operating Frequency |
| 2. | 145.71 | 3 to 33 Hz | Rotation about Y Direction & X Direction & Rotation about Z Direction | Not in or near operating Frequency |
| 3. | 173.41 | 3 to 33 Hz | Rotation about X | Not in or near operating Frequency |
| 4. | 188.83 | 3 to 33 Hz | Z Direction | Not in or near operating Frequency |
| 5. | 258.18 | 3 to 33 Hz | Y Direction | Not in or near operating Frequency |
| 6. | 267.52 | 3 to 33 Hz | Rotation about Y | Not in or near operating Frequency |

## Load Case - 3 Static Structural Analysis

The Finite Element (FE) Static Structural analysis is conducted with the primary objective of assessing the structural integrity and strength of the valve under actual operating loads while the valve is in closed condition. This comprehensive analysis accounts for several critical loads that act upon the various components of the valve during its operational phases. The analysis aims to precisely evaluate the stresses experienced by these valve components, and subsequently, these results will be compared against the allowable limits stipulated for the materials used in the valve's construction.

### Static Structural Analysis Boundary Conditions

The boundary conditions for the finite element structural analysis have been established such that all degrees of freedom are rigidly constrained at both the Inlet and Outlet faces of the Valve, as illustrated in the Figure below.

Figure 35 - Static Structural Analysis: Boundary Conditions Static Structural Analysis

Table 15 - Static Structural Boundary Conditions

| Sl. No. | Loading or BC | Location | Value |
| --- | --- | --- | --- |
| 1 | Remote Displacement (Rotation of the Spindle is Arrested) | Disc | Ux=Uy=Uz=Rx =Rz=Free Ry=0 |
| 2 | Fixed | Pipe End | Ux=Uy=Uz=Rx=Ry=Rz=0 |

Table 16 - Boundary Conditions Summary - Static Structural

| Sr. No | Name of BC | Location of BC | Value of BC |
| --- | --- | --- | --- |
| 1 | Fixed Support | Outlet Pipe | -- |
| 2 | Standard Earth Gravity | Negative Z Axis | -- |
| 3 | Pressure | Wet Surface | 4 [MPa] |
| 4 | Moment | 28 Hand Wheel | 19600 [N mm] |
| 5 | Force (From MBD) | 20 Pinion Shaft | 323.37 [N] |
| 6 | Joint – Force (from MBD) | Joint Load | -25.211 [N] |
| 7 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 8 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 9 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 10 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 11 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 12 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 13 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 14 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 15 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 16 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 17 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 18 | Bolt_Body Bonnet Connections | -- | 13849.18 [N] |
| 19 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 20 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 21 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 22 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 23 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 24 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 25 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 26 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 27 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 28 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 29 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 30 | Bolt_Outlet Flange Connections | -- | 9774.8 [N] |
| 31 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 32 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 33 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 34 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 35 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 36 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 37 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 38 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 39 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 40 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 41 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |
| 42 | Bolt_Inlet Flange Connections | -- | 9774.8 [N] |

### Static Structural Analysis Loading Conditions

### Static Structural Analysis Results

Figure 36 - Static Structural: Total Deformation - End Time

Figure 37 - Static Structural: Equivalent (von-Mises) Stress - End Time

Figure 5 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time

The computed maximum stress of 77.82 MPa is found to be below the permissible stress of 255.71 MPa, confirming that the component will perform reliably under the given loading.

Figure 6 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time

The stress distribution shows a peak value of 141.124 MPa at the most critically loaded region, which is within the acceptable limit of 255.71 MPa. The design is deemed satisfactory.

Figure 7 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time

From the FEA results, the maximum stress developed is 29.007 MPa against an allowable Yield of 84 MPa. Hence the component is safe.

Figure 8 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time

The computed maximum stress of 5.885 MPa is found to be below the permissible stress of 199 MPa, confirming that the component will perform reliably under the given loading.

Figure 9 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time

Analysis results confirm that the highest stress value of 16.592 MPa does not breach the allowable threshold of 185.71 MPa. The component meets the design safety criteria.

Figure 10 - Static Structural: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time

The maximum equivalent stress observed is 41.037 MPa, which is within the allowable limit of 286 MPa. The design is therefore considered safe under the applied loading conditions.

Figure 11 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time

The equivalent stress contour reveals a maximum value of 2.055 MPa, which is less than the allowable limit of 136.67 MPa. Hence the component is structurally adequate.

Figure 12 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time

The stress distribution shows a peak value of 4.003 MPa at the most critically loaded region, which is within the acceptable limit of 177.14 MPa. The design is deemed satisfactory.

Figure 13 - Static Structural: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time

The equivalent stress contour reveals a maximum value of 0.861 MPa, which is less than the allowable limit of 136.67 MPa. Hence the component is structurally adequate.

Figure 14 - Static Structural: All - Force Reaction - Fixed Support - End Time

Table 1 - All - Force Reaction - Fixed Support - End Time - Tabular Data

| Time (S) | Reaction In X Direction (N) | Reaction In Y Direction (N) | Reaction In Z Direction (N) | Total Reaction (N) |
| --- | --- | --- | --- | --- |
| 0.2 | 1.1119e-006 | -3.0263e-006 | 346.85 | 346.85 |
| 0.4 | 2.2158e-006 | -6.0523e-006 | 693.69 | 693.69 |
| 0.7 | 3.8776e-006 | -1.0592e-005 | 1214. | 1214. |
| 1. | 5.5394e-006 | -1.5131e-005 | 1734.2 | 1734.2 |
| 1.2 | -13210 | -10843 | 2049.9 | 17213 |
| 1.4 | -26421 | -21687 | 2365.6 | 34263 |
| 1.7 | -46236 | -37952 | 2839. | 59885 |
| 2. | -66052 | -54217 | 3312.5 | 85518 |

Figure 15 - Static Structural: All - Moment Reaction - Fixed Support - End Time

Table 2 - All - Moment Reaction - Fixed Support - End Time - Tabular Data

| Time (S) | Reaction In X Direction (N) | Reaction In Y Direction (N) | Reaction In Z Direction (N) | Total Reaction (N) |
| --- | --- | --- | --- | --- |
| 0.2 | 1.774e+005 | -1.4315e+005 | -1.9917e-004 | 2.2795e+005 |
| 0.4 | 3.5479e+005 | -2.863e+005 | -4.0247e-004 | 4.559e+005 |
| 0.7 | 6.2088e+005 | -5.0103e+005 | -6.5965e-004 | 7.9783e+005 |
| 1. | 8.8698e+005 | -7.1575e+005 | -1.0574e-003 | 1.1398e+006 |
| 1.2 | 1.0725e+006 | -8.6826e+005 | 8.8638e+005 | 1.6401e+006 |
| 1.4 | 1.2581e+006 | -1.0208e+006 | 1.7728e+006 | 2.4015e+006 |
| 1.7 | 1.5364e+006 | -1.2495e+006 | 3.1023e+006 | 3.6805e+006 |
| 2. | 1.8147e+006 | -1.4783e+006 | 4.4319e+006 | 5.012e+006 |

Table 19 - Bolt Force Summary Table

| Bolt No | Bolt Connection Group | Max Axial Force (N) | Max Shear Force (N) | Bolt Area (mm2) | Normal Stress (MPa) | Shear Stress (MPa) | Yield Stress MPa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 Body To 14 Bonnet | 13966 | 115.61 | 368.3 | 37.9202 | 0.3139 | 825 |
| 2 | 1 Body To 14 Bonnet | 13849 | 116.7 | 368.3 | 37.6025 | 0.3169 | 825 |
| 3 | 1 Body To 14 Bonnet | 13849 | 115.07 | 368.3 | 37.6025 | 0.3124 | 825 |
| 4 | 1 Body To 14 Bonnet | 13849 | 144.54 | 368.3 | 37.6025 | 0.3925 | 825 |
| 5 | 1 Body To 14 Bonnet | 14067 | 133.13 | 368.3 | 38.1944 | 0.3615 | 825 |
| 6 | 1 Body To 14 Bonnet | 14020 | 134.7 | 368.3 | 38.0668 | 0.3657 | 825 |
| 7 | 1 Body To 14 Bonnet | 13849 | 117.76 | 368.3 | 37.6025 | 0.3197 | 825 |
| 8 | 1 Body To 14 Bonnet | 13849 | 59.211 | 368.3 | 37.6025 | 0.1608 | 825 |
| 9 | 1 Body To 14 Bonnet | 13849 | 47.784 | 368.3 | 37.6025 | 0.1297 | 825 |
| 10 | 1 Body To 14 Bonnet | 13887 | 98.114 | 368.3 | 37.7057 | 0.2664 | 825 |
| 11 | 1 Body To 14 Bonnet | 13956 | 109.2 | 368.3 | 37.893 | 0.2965 | 825 |
| 12 | 1 Body To 14 Bonnet | 13996 | 114.79 | 368.3 | 38.0016 | 0.3117 | 825 |
| 13 | Outlet Flange To 1 Body | 9774.8 | 105.04 | 303.4 | 32.2175 | 0.3462 | 825 |
| 14 | Outlet Flange To 1 Body | 9774.8 | 64.796 | 303.4 | 32.2175 | 0.2136 | 825 |
| 15 | Outlet Flange To 1 Body | 9774.8 | 146.35 | 303.4 | 32.2175 | 0.4824 | 825 |
| 16 | Outlet Flange To 1 Body | 9774.8 | 280.29 | 303.4 | 32.2175 | 0.9238 | 825 |
| 17 | Outlet Flange To 1 Body | 9774.8 | 339.27 | 303.4 | 32.2175 | 1.1182 | 825 |
| 18 | Outlet Flange To 1 Body | 9774.8 | 261.76 | 303.4 | 32.2175 | 0.8628 | 825 |
| 19 | Outlet Flange To 1 Body | 9774.8 | 108.06 | 303.4 | 32.2175 | 0.3562 | 825 |
| 20 | Outlet Flange To 1 Body | 9774.8 | 43.161 | 303.4 | 32.2175 | 0.1423 | 825 |
| 21 | Outlet Flange To 1 Body | 9774.8 | 60.867 | 303.4 | 32.2175 | 0.2006 | 825 |
| 22 | Outlet Flange To 1 Body | 9774.8 | 269.72 | 303.4 | 32.2175 | 0.889 | 825 |
| 23 | Outlet Flange To 1 Body | 9774.8 | 217.06 | 303.4 | 32.2175 | 0.7154 | 825 |
| 24 | Outlet Flange To 1 Body | 9774.8 | 196.55 | 303.4 | 32.2175 | 0.6478 | 825 |
| 25 | Inlet Flange To 1 Body | 9774.8 | 123.2 | 303.4 | 32.2175 | 0.4061 | 825 |
| 26 | Inlet Flange To 1 Body | 9774.8 | 122.02 | 303.4 | 32.2175 | 0.4022 | 825 |
| 27 | Inlet Flange To 1 Body | 9774.8 | 71.197 | 303.4 | 32.2175 | 0.2347 | 825 |
| 28 | Inlet Flange To 1 Body | 9774.8 | 95.355 | 303.4 | 32.2175 | 0.3143 | 825 |
| 29 | Inlet Flange To 1 Body | 9774.8 | 147.76 | 303.4 | 32.2175 | 0.487 | 825 |
| 30 | Inlet Flange To 1 Body | 9774.8 | 191.8 | 303.4 | 32.2175 | 0.6322 | 825 |
| 31 | Inlet Flange To 1 Body | 9774.8 | 210.05 | 303.4 | 32.2175 | 0.6923 | 825 |
| 32 | Inlet Flange To 1 Body | 9774.8 | 200.1 | 303.4 | 32.2175 | 0.6595 | 825 |
| 33 | Inlet Flange To 1 Body | 9774.8 | 171.59 | 303.4 | 32.2175 | 0.5656 | 825 |
| 34 | Inlet Flange To 1 Body | 9774.8 | 118.01 | 303.4 | 32.2175 | 0.389 | 825 |
| 35 | Inlet Flange To 1 Body | 9774.8 | 108.18 | 303.4 | 32.2175 | 0.3566 | 825 |
| 36 | Inlet Flange To 1 Body | 9774.8 | 142.5 | 303.4 | 32.2175 | 0.4697 | 825 |
| 37 | 14 Bonnet To 25 Top Cover | -0.0017 | 0.063 | 20.1 | -8.70E-05 | 0.0031 | 320 |
| 38 | 14 Bonnet To 25 Top Cover | 0.0034 | 0.024 | 20.1 | 0.0002 | 0.0012 | 320 |
| 39 | 14 Bonnet To 25 Top Cover | 0.1769 | 0.0744 | 20.1 | 0.0088 | 0.0037 | 320 |
| 40 | 14 Bonnet To 25 Top Cover | 0.0232 | 0.0523 | 20.1 | 0.0012 | 0.0026 | 320 |
| 41 | 14 Bonnet To 25 Top Cover | 0.0598 | 0.0597 | 20.1 | 0.003 | 0.003 | 320 |
| 42 | 14 Bonnet To 25 Top Cover | -0.0118 | 0.0855 | 20.1 | -0.0006 | 0.0043 | 320 |
| 43 | 14 Bonnet To 25 Top Cover | -0.0292 | 0.114 | 20.1 | -0.0015 | 0.0057 | 320 |
| 44 | 14 Bonnet To 25 Top Cover | 0.3708 | 0.1675 | 20.1 | 0.0184 | 0.0083 | 320 |
| 45 | 14 Bonnet To 25 Top Cover | 0.1327 | 0.113 | 20.1 | 0.0066 | 0.0056 | 320 |
| 46 | 14 Bonnet To 25 Top Cover | 0.1488 | 0.0669 | 20.1 | 0.0074 | 0.0033 | 320 |
| 47 | 14 Bonnet To 25 Top Cover | -0.0086 | 0.0596 | 20.1 | -0.0004 | 0.003 | 320 |
| 48 | 14 Bonnet To 32 Side Cover | 0.0411 | 0.0392 | 20.1 | 0.002 | 0.0019 | 320 |
| 49 | 14 Bonnet To 32 Side Cover | -0.1386 | 0.1265 | 20.1 | -0.0069 | 0.0063 | 320 |
| 50 | 14 Bonnet To 32 Side Cover | -0.0082 | 0.0401 | 20.1 | -0.0004 | 0.002 | 320 |
| 51 | 14 Bonnet To 32 Side Cover | 0.6798 | 0.1238 | 20.1 | 0.0338 | 0.0062 | 320 |

### Static Structural Analysis Conclusions

Table 20 - Static Structural Results Summary

| Sr. No | Material | Maximum Occurs On | Stresses Observed(MPa) | Allowable Stress(MPa) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | ASTM B 367 Gr.5 | 1 Body | 77.82 | 255.71 | Stresses less than allowable |
| 2 | ASTM B 348 Gr.5 | 2 Disc | 141.124 | 255.71 | Stresses less than allowable |
| 3 | ASTM D4066- Nylon 66 | 3 Disc Seal | 29.007 | 84 | Stresses less than allowable |
| 4 | BS 970 EN19 | 5 Key | 5.885 | 199 | Stresses less than allowable |
| 5 | BS 2874 CA 104 | 12 Bush-1 | 16.592 | 185.71 | Stresses less than allowable |
| 6 | BS 970 EN36C | 19 Gear | 41.037 | 256.0 | Stresses less than allowable |
| 7 | ASTM A 240 S32100 | 22 Ring Nut | 2.055 | 136.67 | Stresses less than allowable |
| 8 | ASTM A 217 CA15 | 28 Hand Wheel | 4.003 | 177.14 | Stresses less than allowable |
| 9 | ASTM A 240 S32100 | 32 Side Cover | 0.861 | 136.67 | Stresses less than allowable |

## Load case - 4 Vibration resistance analysis

FE Vibration Resistance analysis is a dynamic response analysis of structure. It aims to simulate the response of a structure under propeller induced vibrations. The vibration Resistance tests consist of following four types of tests.

a)	Test for Detection of Resonance Frequencies:

The test is carried out at frequencies ranging from 3-33 Hz with amplitude of oscillations of platform 1.75-0.16 mm. Test frequencies of sinusoidal vibrations are varied gradually upwards or downwards in the entire frequency range (rate of variation of frequency is not more than 5 Hz/min) and at resonant frequency of equipment the response vibration amplitude of the equipment would be maximum and corresponding frequency will be the first Resonance frequency. There could be multiple resonance frequencies in the frequency range of 3-33 Hz. However, the frequency with highest amplitude would be considered for Test.

b)	Vibration Proof Characteristics Test:

This test shall be carried out by gradually increasing the frequency of oscillations in each sub range (refer Table below) from lower to higher frequency and vice versa.

Table 21 - Vibration Proof Characteristics

| Frequency Range | Vibration Amplitude |  | Duration for Vibration Resistance in min | Duration for Vibration Proof Characteristic Test in min |
| --- | --- | --- | --- | --- |
|  | Amplitude Test Platform / Table in (mm) | Acceleration of Test Platform / Table in (g) |  |  |
| Upto 10 Hz Incl. | 1.75 | 0.7 | 24 | >2 |
| Above 10 - Upto 20 Hz Incl. | 0.44 | 0.7 | 24 | >2 |
| Above 20 - Upto 25 Hz Incl. | 0.28 | 0.7 | 24 | >2 |
| Above 25 - Upto 30 Hz Incl. | 0.2 | 0.7 | 24 | >2 |
| Above 30 -Upto 33 Hz Incl. | 0.16 | 0.7 | 24 | >2 |

Here, the given amplitude of oscillations or vibro-acceleration is maintained. The duration of the effect of vibration in each sub-range of frequencies should be adequate for checking parameters, characteristics of the equipment, but not less than 2 minutes. While testing for vibration proof characteristics equipment should be made to perform its functions. For the propulsion motors and rotating machinery, they may be operated on idle run and for Heat Exchangers and fittings the flow of working liquid through the equipment may be stopped, but the pressure of the liquid in the cavities of the equipment should correspond to its working state.

With the equipment performing its functions, test is carried out only when the equipment is in the normal installed position in directions technically feasible on a vibration shaker test stand. In the remaining directions the test is carried out without the equipment performing its functions.

### Vibration Resistance Analysis Methodology

FE Vibration Resistance analysis is to study how a structure or system responds to harmonic (periodic) excitations or loads. It is commonly employed in engineering and physics to understand how structures and components behave under cyclic loads, such as vibrations or oscillations. Analysis has been carried out by mode superposition method.

### Vibration Resistance Analysis Boundary Conditions

As we the Mode Superposition methodology is used the boundary conditions of modal analysis remains same in the Vibration Resistance analysis.

### Vibration Resistance Analysis Loading Conditions

#### Load Case - 3.1 Vibration Resistance Analysis in X- Direction

Figure 49 - Harmonic Response X: Boundary Conditions

Figure 50 - Harmonic Response X: Displacement Amplitude (X)

Table 22 - Displacement Amplitude (X)

| Frequency [Hz] | Magnitude [mm] | Phase Angle [deg] |
| --- | --- | --- |
| 0. | 0. | 0. |
| 3. | 1.75 | 0. |
| 10. | 1.75 | 0. |
| 10.01 | 0.44 | 0. |
| 20. | 0.44 | 0. |
| 20.01 | 0.28 | 0. |
| 25. | 0.28 | 0. |
| 25.01 | 0.2 | 0. |
| 30. | 0.2 | 0. |
| 30.01 | 0.16 | 0. |
| 33. | 0.16 | 0. |

Table 23 - Vibration Resistance Analysis Settings (X)

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Type | Displacement |
| Base Excitation | Yes |
| Absolute Result | Yes |
| Define By | Magnitude - Phase |
| Magnitude | Tabular Data |
| Phase Angle | Tabular Data |
| Direction | X Axis |
| Suppressed | No |

#### Load Case - 3.2 Vibration Resistance Analysis in Y- Direction

Figure 51 - Harmonic Response Y: Boundary Conditions

Figure 52 - Harmonic Response Y: Displacement Amplitude (Y)

Table 24 - Displacement Amplitude (Y)

| Frequency [Hz] | Magnitude [mm] | Phase Angle [deg] |
| --- | --- | --- |
| 0. | 0. | 0. |
| 3. | 1.75 | 0. |
| 10. | 1.75 | 0. |
| 10.01 | 0.44 | 0. |
| 20. | 0.44 | 0. |
| 20.01 | 0.28 | 0. |
| 25. | 0.28 | 0. |
| 25.01 | 0.2 | 0. |
| 30. | 0.2 | 0. |
| 30.01 | 0.16 | 0. |
| 33. | 0.16 | 0. |

Table 25 - Vibration Resistance Analysis Settings (Y)

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Type | Displacement |
| Base Excitation | Yes |
| Absolute Result | Yes |
| Define By | Magnitude - Phase |
| Magnitude | Tabular Data |
| Phase Angle | Tabular Data |
| Direction | Y Axis |
| Suppressed | No |

#### Load Case - 3.3 Vibration Resistance Analysis in Z- Direction

Figure 53 - Harmonic Response Z: Boundary Conditions

Figure 54 - Harmonic Response Z: Displacement Amplitude (Z)

Table 26 - Displacement Amplitude (Z)

| Frequency [Hz] | Magnitude [mm] | Phase Angle [deg] |
| --- | --- | --- |
| 0. | 0. | 0. |
| 3. | 1.75 | 0. |
| 10. | 1.75 | 0. |
| 10.01 | 0.44 | 0. |
| 20. | 0.44 | 0. |
| 20.01 | 0.28 | 0. |
| 25. | 0.28 | 0. |
| 25.01 | 0.2 | 0. |
| 30. | 0.2 | 0. |
| 30.01 | 0.16 | 0. |
| 33. | 0.16 | 0. |

Table 27 - Vibration Resistance Analysis Settings (Z)

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Type | Displacement |
| Base Excitation | Yes |
| Absolute Result | Yes |
| Define By | Magnitude - Phase |
| Magnitude | Tabular Data |
| Phase Angle | Tabular Data |
| Direction | Z Axis |
| Suppressed | No |

### Vibration Resistance analysis Results

#### Analysis Harmonic In X direction

Figure 16 - Harmonic Response X: Equivalent (von-Mises) Stress

Figure 17 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5

Figure 18 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5

Figure 19 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66

Figure 20 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS970_EN19

Figure 21 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS2874_CA104

Figure 22 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_BS970_EN36C

Figure 23 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100

Figure 24 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15

Figure 25 - Harmonic Response X: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100

Figure 26 - Harmonic Response X: Frequency Response_MAT_ASTM_B367_Gr_5

Figure 27 - Harmonic Response X: Frequency Response_MAT_ASTM_B348_Gr_5

Figure 28 - Harmonic Response X: Frequency Response_MAT_ASTM_D4066_Nylon66

Figure 29 - Harmonic Response X: Frequency Response_MAT_BS970_EN19

Figure 30 - Harmonic Response X: Frequency Response_MAT_BS2874_CA104

Figure 31 - Harmonic Response X: Frequency Response_MAT_BS970_EN36C

Figure 32 - Harmonic Response X: Frequency Response_MAT_ASTM_A276_S32100

Figure 33 - Harmonic Response X: Frequency Response_MAT_ASTM_A217_CA15

Figure 34 - Harmonic Response X: Frequency Response_MAT_ASTM_A240_S32100

#### Analysis Harmonic In Y direction

Figure 35 - Harmonic Response Y: Equivalent (von-Mises) Stress

Figure 36 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5

Figure 37 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5

Figure 38 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66

Figure 39 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS970_EN19

Figure 40 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS2874_CA104

Figure 41 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_BS970_EN36C

Figure 42 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100

Figure 43 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15

Figure 44 - Harmonic Response Y: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100

Figure 45 - Harmonic Response Y: Frequency Response_MAT_ASTM_B367_Gr_5

Figure 46 - Harmonic Response Y: Frequency Response_MAT_ASTM_B348_Gr_5

Figure 47 - Harmonic Response Y: Frequency Response_MAT_ASTM_D4066_Nylon66

Figure 48 - Harmonic Response Y: Frequency Response_MAT_BS970_EN19

Figure 49 - Harmonic Response Y: Frequency Response_MAT_BS2874_CA104

Figure 50 - Harmonic Response Y: Frequency Response_MAT_BS970_EN36C

Figure 51 - Harmonic Response Y: Frequency Response_MAT_ASTM_A276_S32100

Figure 52 - Harmonic Response Y: Frequency Response_MAT_ASTM_A217_CA15

Figure 53 - Harmonic Response Y: Frequency Response_MAT_ASTM_A240_S32100

#### Analysis Harmonic In Z direction

Figure 54 - Harmonic Response Z: Equivalent (von-Mises) Stress

Figure 55 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5

Figure 56 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5

Figure 57 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66

Figure 58 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS970_EN19

Figure 59 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS2874_CA104

Figure 60 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_BS970_EN36C

Figure 61 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100

Figure 62 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15

Figure 63 - Harmonic Response Z: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100

Figure 64 - Harmonic Response Z: Frequency Response_MAT_ASTM_B367_Gr_5

Figure 65 - Harmonic Response Z: Frequency Response_MAT_ASTM_B348_Gr_5

Figure 66 - Harmonic Response Z: Frequency Response_MAT_ASTM_D4066_Nylon66

Figure 67 - Harmonic Response Z: Frequency Response_MAT_BS970_EN19

Figure 68 - Harmonic Response Z: Frequency Response_MAT_BS2874_CA104

Figure 69 - Harmonic Response Z: Frequency Response_MAT_BS970_EN36C

Figure 70 - Harmonic Response Z: Frequency Response_MAT_ASTM_A276_S32100

Figure 71 - Harmonic Response Z: Frequency Response_MAT_ASTM_A217_CA15

Figure 72 - Harmonic Response Z: Frequency Response_MAT_ASTM_A240_S32100

### Mapping of materials to the Cases as per

- Case (a): Carbon & Low Alloy Steels

- Case (b): Series 3XX Stainless Steels & Nickel Alloys

- Case (c): Wrought 70–30 Copper–Nickel

- Case (d): Ni–Cr–Mo–Fe Alloys

- Case (e): High Strength Bolting

Table 26 Material Case Mapping according to ANNEX 3-F DESIGN FATIGUE CURVES of ASME BPVC.VIII.2-2023

| Material | UNS | Type | Case |
| --- | --- | --- | --- |
| ASTM A217 CA15 | J91150 | Ferrous | a (martensitic carbon/low alloy steel) |
| BS 970 EN19 | G41400 | Ferrous | a (martensitic carbon/low alloy steel) |
| BS 970 EN36C | G93100 | Ferrous | a (martensitic carbon/low alloy steel) |
| ASTM A 276 S32100 | S32100 | Ferrous | b (3XX stainless steel) |
| ASTM A 240 S32100 | S32100 | Ferrous | b (3XX stainless steel) |
| BS 2874 CA104 | C63000 | Non-Ferrous | NAB |
| ASTM B 367 Gr.5 | R56400 | Non-Ferrous | Ti Gr 5 |
| ASTM B 348 Gr.5 | R56400 | Non-Ferrous | Ti Gr 5 |

### Harmonic Analysis Conclusions

Table 28 - Static Structural Results Summary

| Sr. No. | Direction | Material | Maximum Occurs On | Stresses Observed (MPa) | Allowable Stress (MPa) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Along X Axis | ASTM B 367 Gr.5 | 1 Body | 2.1826 | 237.5560851 | Stresses less than allowable |
| 2 | Along X Axis | ASTM B 348 Gr.5 | 4 Piston | 11.99 | 237.5560851 | Stresses less than allowable |
| 3 | Along X Axis | ASTM D4066- Nylon 66 | 3 Disc Seal | 0.15569 | Refer Note 1 |  |
| 4 | Along X Axis | BS 970 EN19 | 5 Key | 0.63141 | 57.21082297 | Stresses less than allowable |
| 5 | Along X Axis | BS 2874 CA 104 | 12 Bush-1 | 1.0756 | 59.55630959 | Stresses less than allowable |
| 6 | Along X Axis | BS 970 EN36C | 20 Pinion Shaft | 0.84086 | 52.61616193 | Stresses less than allowable |
| 7 | Along X Axis | ASTM A 276 S32100 | 22 Ring Nut | 0.046613 | 65.1775787 | Stresses less than allowable |
| 8 | Along X Axis | ASTM A 217 CA15 | 28 Hand Wheel | 0.31959 | 52.86741182 | Stresses less than allowable |
| 9 | Along X Axis | ASTM A 240 S32100 | 32 Side Cover | 1.2804 | 65.1775787 | Stresses less than allowable |
| 10 | Along Y Axis | ASTM B 367 Gr.5 | 1 Body | 2.3849 | 237.5560851 | Stresses less than allowable |
| 11 | Along Y Axis | ASTM B 348 Gr.5 | 4 Piston | 9.3175 | 237.5560851 | Stresses less than allowable |
| 12 | Along Y Axis | ASTM D4066- Nylon 66 | 3 Disc Seal | 0.09562 | Refer Note 1 | Stresses less than allowable |
| 13 | Along Y Axis | BS 970 EN19 | 5 Key | 0.39067 | 57.21082297 | Stresses less than allowable |
| 14 | Along Y Axis | BS 2874 CA 104 | 15 Yoke Sleeve | 0.364576 | 59.55630959 | Stresses less than allowable |
| 15 | Along Y Axis | BS 970 EN36C | 19 Gear | 0.12565 | 52.61616193 | Stresses less than allowable |
| 16 | Along Y Axis | ASTM A 276 S32100 | 22 Ring Nut | 0.0476 | 65.1775787 | Stresses less than allowable |
| 17 | Along Y Axis | ASTM A 217 CA15 | 28 Handwheel | 0.3516 | 52.86741182 | Stresses less than allowable |
| 18 | Along Y Axis | ASTM A 240 S32100 | 32 Side Cover | 0.70264 | 65.1775787 | Stresses less than allowable |
| 19 | Along Z Axis | ASTM B 367 Gr.5 | 1 Body | 8.5785 | 237.5560851 | Stresses less than allowable |
| 20 | Along Z Axis | ASTM B 348 Gr.5 | 4 Piston | 13.051 | 237.5560851 | Stresses less than allowable |
| 21 | Along Z Axis | ASTM D4066- Nylon 66 | 3 Disc Seal | 1.63 | Refer Note 1 | Stresses less than allowable |
| 22 | Along Z Axis | BS 970 EN19 | 5 Key | 2.6166 | 57.21082297 | Stresses less than allowable |
| 23 | Along Z Axis | BS 2874 CA 104 | 12 Bush -1 | 10.295 | 59.55630959 | Stresses less than allowable |
| 24 | Along Z Axis | BS 970 EN36C | 20 Pinion Shaft | 8.3706 | 52.61616193 | Stresses less than allowable |
| 25 | Along Z Axis | ASTM A 276 S32100 | 22 Ring Nut | 0.31474 | 65.1775787 | Stresses less than allowable |
| 26 | Along Z Axis | ASTM A 217 CA15 | 28 Handwheel | 3.0835 | 52.86741182 | Stresses less than allowable |
| 27 | Along Z Axis | ASTM A 240 S32100 | 32 Side Cover | 4.8676 | 65.1775787 | Stresses less than allowable |

Note 1: Reliable fatigue curves for ASTM D4066 Nylon 66 are not available. Additionally, the gaskets and seals are bought-out items and are classified as consumables. Extra quantities of these items will be supplied as onboard spares.

(a)

(b)

Figure 74 Martensitic carbon/low alloy steel fatigue curve (ASME BPVC.VIII.2-2023, Page 161,162)

Table 28 Allowable calculations for the Martensitic carbon/low alloy steel

| Material | UTS | Stress at at 10^7 Cycles | Surface Roughness Factor | Fatigue Allowable |
| --- | --- | --- | --- | --- |
| ASTM A217 CA15 | 620 | 92.67219917 | 0.570477579 | 52.86741179 |
| BS 970 EN36C | 1000 | 130 | 0.404739707 | 52.61616193 |
| BS 970 EN19 | 695 | 108.8547718 | 0.525570189 | 57.21082297 |

The Surface Finish Factor using equation (Chapter 6, Equation 6.7 e, Pg. 329) is calculated. Machine Design - An Integrated Approach by Robert L. Norton.

Here A= 57.7 and b= -0.718 are used though the minimum surface finish for any of the component will be as machined.

- Roshan et al

- Sarkar et al

(c) Taylor and Knott 1982

Figure 75 NAB Fatigue Curves

Table 29 Allowable calculations for NAB

| Material | Surface Roughness Factor |  | Fatigue Allowable |
| --- | --- | --- | --- |
|  |  | Page 80 |  |
|  |  | Figure 75 (a) of this report |  |
| BS2874 CA104 | 0.551447311 | 108 | 59.55630959 |

(a) Kikuchi et al

(b) Siatova et al

(c) Zherebtsov et al

Figure 76 Ti Gr 5 Fatigue Curves

Table 30 Allowable Calculations for Ti Gr 5

| Material | Surface Roughness Factor | Page 81 of this Report | Fatigue Allowable |
| --- | --- | --- | --- |
|  |  | Figure 76 (b) |  |
| ASTM B 367 Gr.5 | 0.43829536 | 542 | 237.5560851 |
| ASTM B 348 Gr.5 | 0.43829536 | 542 | 237.5560851 |

## Load Case - 5 Shock Analysis - Transient Approach

Shock analysis has been conducted to replicate and evaluate the structural integrity of the valve under the dynamic shock loads which experiences when subjected to the K-200 Shock testing machine.

For shock analysis, two distinct methodologies are available: the transient shock approach and the Equivalent Static approach. The selection between these approaches is based on critical parameter namely, the first natural frequency of the structure as determined in the modal analysis.

In this specific scenario, the first natural frequency of the valve established through modal analysis is above the threshold of 160 Hz. Consequently, the chosen methodology for simulating and estimating the shock strength of the valve is the Equivalent Static Approach. This approach effectively captures and quantifies the valve's ability to withstand and respond to the shock loads it experiences during its operation on the K-200 Shock testing machine. Shock analysis is performed in 3 directions of shock loads i.e., X, Y and Z direction.

### Shock Analysis Boundary And Loading Conditions For Transient Analysis

Figure 109 - Transient Analysis: Boundary Conditions

Figure 110 - Transient: Transient - Shock Load

Table 29 - Analysis Settings

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Base Excitation | Yes |
| Absolute Result | No |
| Magnitude | Tabular Data |
| Direction | X Axis |
| Suppressed | No |

Figure 111 - Transient Analysis: Boundary Conditions

Figure 112 - Transient: Transient - Shock Load

Table 30 - Analysis Settings

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Base Excitation | Yes |
| Absolute Result | No |
| Magnitude | Tabular Data |
| Direction | Y Axis |
| Suppressed | No |

Figure 113 - Transient Analysis: Boundary Conditions

Figure 114 - Transient: Transient - Shock Load

Table 31 - Analysis Settings

| Property | Value |
| --- | --- |
| Boundary Condition | Fixed Support |
| Base Excitation | Yes |
| Absolute Result | No |
| Magnitude | Tabular Data |
| Direction | Z Axis |
| Suppressed | No |

### Shock Pulse Specifications

Figure 115 - Shock Analysis: Shock Pulse

Table 32 - Shock Pulse Specification Table

|  | Vertical |  |  |  | Longitudinal |  |  |  | Horizontal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weight (Kg) upto | A1 | T1 | A2 | T2 | A1 | T1 | A2 | T2 | A1 | T1 | A2 | T2 |
|  | g | ms | g | ms | g | ms | g | ms | g | ms | g | ms |
| 60 | 180 | 3 | 83.5 | 6.5 | 220 | 2.5 | 59.5 | 9 | 280 | 3 | 94 | 9 |
| 140 | 170 | 2 | 36 | 9.5 | 245 | 2.5 | 54 | 11.5 | 300 | 2 | 34 | 17.5 |
| 200 | 130 | 2.5 | 35 | 9 | 280 | 2.5 | 90.5 | 7.5 | 280 | 2 | 34 | 16.5 |

### Results of Shock Analysis

#### Analysis Transient in X Direction

Figure 116 - Transient X: Total Deformation - End Time

Figure 117 - Transient X: Equivalent (von-Mises) Stress - End Time

Figure 75 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time

The stress distribution shows a peak value of 77.317 MPa at the most critically loaded region, which is within the Static Allowable limit of 255.71 MPa. The design is deemed satisfactory.

Figure 76 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time

The peak Von Mises stress in the component is 52.234 MPa, remaining below the Static Allowable stress of 255.71 MPa. The structural integrity of the component is confirmed.

Figure 77 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time

The stress distribution shows a peak value of 6.882 MPa at the most critically loaded region, which is within the Static Allowable limit of 28.57 MPa. The design is deemed satisfactory.

Figure 78 - Transient X: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time

FE analysis indicates a maximum stress of 36.572 MPa at the critical location, which does not exceed the Static Allowable stress of 199 MPa. The component satisfies the strength requirements.

Figure 79 - Transient X: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time

From the FEA results, the maximum stress developed is 63.33 MPa against an Static Allowable of 185.71 MPa. The factor of safety is adequate and the component is safe.

Figure 80 - Transient X: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time

The computed maximum stress of 52.419 MPa is found to be below the Static Allowable stress of 286 MPa, confirming that the component will perform reliably under the given loading.

Figure 81 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time

From the FEA results, the maximum stress developed is 2.688 MPa against an Static Allowable of 136.67 MPa. The factor of safety is adequate and the component is safe.

Figure 82 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time

The stress distribution shows a peak value of 22.546 MPa at the most critically loaded region, which is within the Static Allowable limit of 177.14 MPa. The design is deemed satisfactory.

Figure 83 - Transient X: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time

FE analysis indicates a maximum stress of 73.224 MPa at the critical location, which does not exceed the Static Allowable stress of 136.67 MPa. The component satisfies the strength requirements.

Table 33 - Bolt Loads: Shock in X direction

| Bolt No | Bolt Connection Group | Max Axial Force (N) | Max Shear Force (N) | Bolt Area (mm2) | Normal Stress (MPa) | Shear Stress (MPa) | Yield Stress MPa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 Body To 14 Bonnet | 96.072 | 95.297 | 368.3 | 0.2609 | 0.2587 | 825 |
| 2 | 1 Body To 14 Bonnet | 89.81 | 111.7 | 368.3 | 0.2439 | 0.3033 | 825 |
| 3 | 1 Body To 14 Bonnet | 367.12 | 64.75 | 368.3 | 0.9968 | 0.1758 | 825 |
| 4 | 1 Body To 14 Bonnet | 200.1 | 11.496 | 368.3 | 0.5433 | 0.0312 | 825 |
| 5 | 1 Body To 14 Bonnet | 43.292 | 31.656 | 368.3 | 0.1175 | 0.086 | 825 |
| 6 | 1 Body To 14 Bonnet | 33.341 | 35.644 | 368.3 | 0.0905 | 0.0968 | 825 |
| 7 | 1 Body To 14 Bonnet | 196.25 | 12.941 | 368.3 | 0.5329 | 0.0351 | 825 |
| 8 | 1 Body To 14 Bonnet | 422.76 | 60.44 | 368.3 | 1.1479 | 0.1641 | 825 |
| 9 | 1 Body To 14 Bonnet | 114.85 | 103.68 | 368.3 | 0.3118 | 0.2815 | 825 |
| 10 | 1 Body To 14 Bonnet | 83.666 | 102.16 | 368.3 | 0.2272 | 0.2774 | 825 |
| 11 | 1 Body To 14 Bonnet | 143.88 | 48.917 | 368.3 | 0.3907 | 0.1328 | 825 |
| 12 | 1 Body To 14 Bonnet | 150.05 | 35.751 | 368.3 | 0.4074 | 0.0971 | 825 |
| 13 | Outlet Flange To 1 Body | 244.89 | 192.62 | 303.4 | 0.8072 | 0.6349 | 825 |
| 14 | Outlet Flange To 1 Body | 320.63 | 226.5 | 303.4 | 1.0568 | 0.7465 | 825 |
| 15 | Outlet Flange To 1 Body | 289.86 | 223.29 | 303.4 | 0.9554 | 0.736 | 825 |
| 16 | Outlet Flange To 1 Body | 323.24 | 209.92 | 303.4 | 1.0654 | 0.6919 | 825 |
| 17 | Outlet Flange To 1 Body | 313.41 | 201.98 | 303.4 | 1.033 | 0.6657 | 825 |
| 18 | Outlet Flange To 1 Body | 213.88 | 180.11 | 303.4 | 0.7049 | 0.5936 | 825 |
| 19 | Outlet Flange To 1 Body | 130.15 | 169.41 | 303.4 | 0.429 | 0.5584 | 825 |
| 20 | Outlet Flange To 1 Body | 83.883 | 165.72 | 303.4 | 0.2765 | 0.5462 | 825 |
| 21 | Outlet Flange To 1 Body | 64.174 | 145.86 | 303.4 | 0.2115 | 0.4808 | 825 |
| 22 | Outlet Flange To 1 Body | 90.79 | 140.36 | 303.4 | 0.2992 | 0.4626 | 825 |
| 23 | Outlet Flange To 1 Body | 149.95 | 153.56 | 303.4 | 0.4942 | 0.5061 | 825 |
| 24 | Outlet Flange To 1 Body | 65.431 | 134.43 | 303.4 | 0.2157 | 0.4431 | 825 |
| 25 | Inlet Flange To 1 Body | 183.94 | 157.45 | 303.4 | 0.6063 | 0.519 | 825 |
| 26 | Inlet Flange To 1 Body | 199.67 | 118.9 | 303.4 | 0.6581 | 0.3919 | 825 |
| 27 | Inlet Flange To 1 Body | 200.79 | 92.97 | 303.4 | 0.6618 | 0.3064 | 825 |
| 28 | Inlet Flange To 1 Body | 171.64 | 104.2 | 303.4 | 0.5657 | 0.3434 | 825 |
| 29 | Inlet Flange To 1 Body | 55.364 | 127.74 | 303.4 | 0.1825 | 0.421 | 825 |
| 30 | Inlet Flange To 1 Body | 206.66 | 149.03 | 303.4 | 0.6811 | 0.4912 | 825 |
| 31 | Inlet Flange To 1 Body | 393.81 | 163.51 | 303.4 | 1.298 | 0.5389 | 825 |
| 32 | Inlet Flange To 1 Body | 506.26 | 172.15 | 303.4 | 1.6686 | 0.5674 | 825 |
| 33 | Inlet Flange To 1 Body | 523.44 | 200.18 | 303.4 | 1.7252 | 0.6598 | 825 |
| 34 | Inlet Flange To 1 Body | 350.52 | 222.79 | 303.4 | 1.1553 | 0.7343 | 825 |
| 35 | Inlet Flange To 1 Body | 107.85 | 194.43 | 303.4 | 0.3555 | 0.6408 | 825 |
| 36 | Inlet Flange To 1 Body | 464.58 | 227.81 | 303.4 | 1.5312 | 0.7509 | 825 |
| 37 | 14 Bonnet To 25 Top Cover | 0.584 | 0.8518 | 20.1 | 0.0291 | 0.0424 | 320 |
| 38 | 14 Bonnet To 25 Top Cover | 9.6448 | 0.2918 | 20.1 | 0.4798 | 0.0145 | 320 |
| 39 | 14 Bonnet To 25 Top Cover | 5.1359 | 1.0333 | 20.1 | 0.2555 | 0.0514 | 320 |
| 40 | 14 Bonnet To 25 Top Cover | 4.0825 | 1.0616 | 20.1 | 0.2031 | 0.0528 | 320 |
| 41 | 14 Bonnet To 25 Top Cover | 0.5756 | 0.8479 | 20.1 | 0.0286 | 0.0422 | 320 |
| 42 | 14 Bonnet To 25 Top Cover | 0.7476 | 1.1651 | 20.1 | 0.0372 | 0.058 | 320 |
| 43 | 14 Bonnet To 25 Top Cover | 1.6967 | 1.2317 | 20.1 | 0.0844 | 0.0613 | 320 |
| 44 | 14 Bonnet To 25 Top Cover | 1.9451 | 1.1065 | 20.1 | 0.0968 | 0.055 | 320 |
| 45 | 14 Bonnet To 25 Top Cover | 0.7294 | 1.1048 | 20.1 | 0.0363 | 0.055 | 320 |
| 46 | 14 Bonnet To 25 Top Cover | 0.4869 | 1.0326 | 20.1 | 0.0242 | 0.0514 | 320 |
| 47 | 14 Bonnet To 25 Top Cover | 0.5813 | 1.0853 | 20.1 | 0.0289 | 0.054 | 320 |
| 48 | 14 Bonnet To 32 Side Cover | 6.2128 | 2.6622 | 20.1 | 0.3091 | 0.1324 | 320 |
| 49 | 14 Bonnet To 32 Side Cover | 0.7721 | 20.927 | 20.1 | 0.0384 | 1.0411 | 320 |
| 50 | 14 Bonnet To 32 Side Cover | 6.5264 | 2.7331 | 20.1 | 0.3247 | 0.136 | 320 |
| 51 | 14 Bonnet To 32 Side Cover | 2.2697 | 20.525 | 20.1 | 0.1129 | 1.0211 | 320 |

#### Analysis Transient in Y Direction

Figure 127 - Transient Y: Total Deformation - End Time

Figure 128 - Transient Y: Equivalent (von-Mises) Stress - End Time

Figure 86 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time

FE analysis indicates a maximum stress of 40.128 MPa at the critical location, which does not exceed the Static Allowable stress of 255.71 MPa. The component satisfies the strength requirements.

Figure 87 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time

The equivalent stress contour reveals a maximum value of 22.371 MPa, which is less than the allowable limit of 255.71 MPa. Hence the component is structurally adequate.

Figure 88 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time

Analysis results confirm that the highest stress value of 4.511 MPa does not breach the Static Allowable threshold of 28.57 MPa. The component meets the design safety criteria.

Figure 89 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time

From the FEA results, the maximum stress developed is 17.59 MPa against an Static Allowable of 199 MPa. The factor of safety is adequate and the component is safe.

Figure 90 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time

The maximum equivalent stress observed is 18.925 MPa, which is within the Static Allowable limit of 185.71 MPa. The design is therefore considered safe under the applied loading conditions.

Figure 91 - Transient Y: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time

The equivalent stress contour reveals a maximum value of 7.405 MPa, which is less than the Static Allowable limit of 286.0 MPa. Hence the component is structurally adequate.

Figure 92 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time

The stress distribution shows a peak value of 2.131 MPa at the most critically loaded region, which is within the Static Allowable limit of 136.67 MPa. The design is deemed satisfactory.

Figure 93 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time

Analysis results confirm that the highest stress value of 15.948 MPa does not breach the Static Allowable threshold of 177.14 MPa. The component meets the design safety criteria.

Figure 94 - Transient Y: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time

FE analysis indicates a maximum stress of 35.963 MPa at the critical location, which does not exceed the Static Allowable stress of 136.67 MPa. The component satisfies the strength requirements.

Table 34 - Bolt Loads: Shock in Y direction

| Bolt No | Bolt Connection Group | Max Axial Force (N) | Max Shear Force (N) | Bolt Area (mm2) | Normal Stress (MPa) | Shear Stress (MPa) | Yield Stress MPa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 Body To 14 Bonnet | 79.923 | 14.839 | 368.3 | 0.217 | 0.0403 | 825 |
| 2 | 1 Body To 14 Bonnet | 131.93 | 14.961 | 368.3 | 0.3582 | 0.0406 | 825 |
| 3 | 1 Body To 14 Bonnet | 190.15 | 7.2786 | 368.3 | 0.5163 | 0.0198 | 825 |
| 4 | 1 Body To 14 Bonnet | 148.4 | 17.953 | 368.3 | 0.4029 | 0.0487 | 825 |
| 5 | 1 Body To 14 Bonnet | 43.338 | 16.361 | 368.3 | 0.1177 | 0.0444 | 825 |
| 6 | 1 Body To 14 Bonnet | 30.067 | 16.882 | 368.3 | 0.0816 | 0.0458 | 825 |
| 7 | 1 Body To 14 Bonnet | 151.53 | 25.105 | 368.3 | 0.4114 | 0.0682 | 825 |
| 8 | 1 Body To 14 Bonnet | 318.45 | 15.34 | 368.3 | 0.8646 | 0.0417 | 825 |
| 9 | 1 Body To 14 Bonnet | 249.14 | 20.765 | 368.3 | 0.6765 | 0.0564 | 825 |
| 10 | 1 Body To 14 Bonnet | 100.2 | 20.283 | 368.3 | 0.2721 | 0.0551 | 825 |
| 11 | 1 Body To 14 Bonnet | 65.802 | 11.441 | 368.3 | 0.1787 | 0.0311 | 825 |
| 12 | 1 Body To 14 Bonnet | 63.271 | 10.076 | 368.3 | 0.1718 | 0.0274 | 825 |
| 13 | Outlet Flange To 1 Body | 50.184 | 268.1 | 303.4 | 0.1654 | 0.8837 | 825 |
| 14 | Outlet Flange To 1 Body | 64.231 | 275.31 | 303.4 | 0.2117 | 0.9074 | 825 |
| 15 | Outlet Flange To 1 Body | 53.955 | 238.52 | 303.4 | 0.1778 | 0.7862 | 825 |
| 16 | Outlet Flange To 1 Body | 56.478 | 210.89 | 303.4 | 0.1862 | 0.6951 | 825 |
| 17 | Outlet Flange To 1 Body | 58.921 | 198.43 | 303.4 | 0.1942 | 0.654 | 825 |
| 18 | Outlet Flange To 1 Body | 43.476 | 179.37 | 303.4 | 0.1433 | 0.5912 | 825 |
| 19 | Outlet Flange To 1 Body | 65.876 | 159.65 | 303.4 | 0.2171 | 0.5262 | 825 |
| 20 | Outlet Flange To 1 Body | 120.68 | 144.82 | 303.4 | 0.3978 | 0.4773 | 825 |
| 21 | Outlet Flange To 1 Body | 124.26 | 136.85 | 303.4 | 0.4096 | 0.4511 | 825 |
| 22 | Outlet Flange To 1 Body | 129.54 | 176.61 | 303.4 | 0.427 | 0.5821 | 825 |
| 23 | Outlet Flange To 1 Body | 93.813 | 226.23 | 303.4 | 0.3092 | 0.7456 | 825 |
| 24 | Outlet Flange To 1 Body | 124.42 | 142.44 | 303.4 | 0.4101 | 0.4695 | 825 |
| 25 | Inlet Flange To 1 Body | 111.43 | 113.54 | 303.4 | 0.3673 | 0.3742 | 825 |
| 26 | Inlet Flange To 1 Body | 85.508 | 108.12 | 303.4 | 0.2818 | 0.3564 | 825 |
| 27 | Inlet Flange To 1 Body | 65.474 | 95.282 | 303.4 | 0.2158 | 0.314 | 825 |
| 28 | Inlet Flange To 1 Body | 72.805 | 104.01 | 303.4 | 0.24 | 0.3428 | 825 |
| 29 | Inlet Flange To 1 Body | 57.801 | 123.11 | 303.4 | 0.1905 | 0.4058 | 825 |
| 30 | Inlet Flange To 1 Body | 56.212 | 144.49 | 303.4 | 0.1853 | 0.4762 | 825 |
| 31 | Inlet Flange To 1 Body | 64.614 | 162.56 | 303.4 | 0.213 | 0.5358 | 825 |
| 32 | Inlet Flange To 1 Body | 66.527 | 173.05 | 303.4 | 0.2193 | 0.5704 | 825 |
| 33 | Inlet Flange To 1 Body | 61.711 | 175.84 | 303.4 | 0.2034 | 0.5796 | 825 |
| 34 | Inlet Flange To 1 Body | 110.4 | 149.27 | 303.4 | 0.3639 | 0.492 | 825 |
| 35 | Inlet Flange To 1 Body | 118.28 | 125.11 | 303.4 | 0.3898 | 0.4124 | 825 |
| 36 | Inlet Flange To 1 Body | 85.525 | 167.94 | 303.4 | 0.2819 | 0.5535 | 825 |
| 37 | 14 Bonnet To 25 Top Cover | 2.5319 | 0.2078 | 20.1 | 0.126 | 0.0103 | 320 |
| 38 | 14 Bonnet To 25 Top Cover | 1.6256 | 0.0431 | 20.1 | 0.0809 | 0.0021 | 320 |
| 39 | 14 Bonnet To 25 Top Cover | 3.2205 | 0.0849 | 20.1 | 0.1602 | 0.0042 | 320 |
| 40 | 14 Bonnet To 25 Top Cover | 2.2699 | 0.1219 | 20.1 | 0.1129 | 0.0061 | 320 |
| 41 | 14 Bonnet To 25 Top Cover | 2.5438 | 0.2229 | 20.1 | 0.1266 | 0.0111 | 320 |
| 42 | 14 Bonnet To 25 Top Cover | 2.6702 | 0.1811 | 20.1 | 0.1328 | 0.009 | 320 |
| 43 | 14 Bonnet To 25 Top Cover | 3.9604 | 0.2013 | 20.1 | 0.197 | 0.01 | 320 |
| 44 | 14 Bonnet To 25 Top Cover | 4.1528 | 0.3003 | 20.1 | 0.2066 | 0.0149 | 320 |
| 45 | 14 Bonnet To 25 Top Cover | 2.8515 | 0.2539 | 20.1 | 0.1419 | 0.0126 | 320 |
| 46 | 14 Bonnet To 25 Top Cover | 2.5705 | 0.2503 | 20.1 | 0.1279 | 0.0125 | 320 |
| 47 | 14 Bonnet To 25 Top Cover | 2.5527 | 0.1832 | 20.1 | 0.127 | 0.0091 | 320 |
| 48 | 14 Bonnet To 32 Side Cover | 1.9212 | 0.2435 | 20.1 | 0.0956 | 0.0121 | 320 |
| 49 | 14 Bonnet To 32 Side Cover | 6.5167 | 7.5373 | 20.1 | 0.3242 | 0.375 | 320 |
| 50 | 14 Bonnet To 32 Side Cover | 1.9822 | 0.2537 | 20.1 | 0.0986 | 0.0126 | 320 |
| 51 | 14 Bonnet To 32 Side Cover | 7.8221 | 7.2081 | 20.1 | 0.3892 | 0.3586 | 320 |

#### Analysis Transient in Z Direction

Figure 138 - Transient Z: Total Deformation - End Time

Figure 139 - Transient Z: Equivalent (von-Mises) Stress - End Time

Figure 97 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_B367_Gr_5 - End Time

The stress distribution shows a peak value of 99.793 MPa at the most critically loaded region, which is within the Static Allowable limit of 255.71 MPa. The design is deemed satisfactory.

Figure 98 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_B348_Gr_5 - End Time

The computed maximum stress of 113.782 MPa is found to be below the Static Allowable of 255.71 MPa, confirming that the component will perform reliably under the given loading.

Figure 99 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_D4066_Nylon66 - End Time

Maximum stress computed from the finite element solution is 17.788 MPa, well within the Static Allowable of 28.57 MPa. No structural failure is anticipated.

Figure 100 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS970_EN19 - End Time

The equivalent stress contour reveals a maximum value of 31.116 MPa, which is less than the Static Allowable limit of 199 MPa. Hence the component is structurally adequate.

Figure 101 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS2874_CA104 - End Time

The stress distribution shows a peak value of 125.832 MPa at the most critically loaded region, which is within the Static Allowable limit of 185.71 MPa. The design is deemed satisfactory.

Figure 102 - Transient Z: Equivalent (von-Mises) Stress - MAT_BS970_EN36C - End Time

The peak Von Mises stress in the component is 105.761 MPa, remaining below the Static Allowable of 286.0 MPa. The structural integrity of the component is confirmed.

Figure 103 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A276_S32100 - End Time

From the FEA results, the maximum stress developed is 5.142 MPa against an Static Allowable of 136.67 MPa. The factor of safety is adequate and the component is safe.

Figure 104 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A217_CA15 - End Time

The stress distribution shows a peak value of 42.92 MPa at the most critically loaded region, which is within the Static Allowable limit of 177.14 MPa. The design is deemed satisfactory.

Figure 105 - Transient Z: Equivalent (von-Mises) Stress - MAT_ASTM_A240_S32100 - End Time

FE analysis indicates a maximum stress of 56.866 MPa at the critical location, which does not exceed the Static Allowable stress of 136.67 MPa. The component satisfies the strength requirements.

Table 35 - Bolt Loads: Shock in Z direction

| Bolt No | Bolt Connection Group | Max Axial Force (N) | Max Shear Force (N) | Bolt Area (mm2) | Normal Stress (MPa) | Shear Stress (MPa) | Yield Stress MPa |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 Body To 14 Bonnet | 525.32 | 74.863 | 368.3 | 1.4263 | 0.2033 | 825 |
| 2 | 1 Body To 14 Bonnet | 614.13 | 14.029 | 368.3 | 1.6675 | 0.0381 | 825 |
| 3 | 1 Body To 14 Bonnet | 398.44 | 80.333 | 368.3 | 1.0818 | 0.2181 | 825 |
| 4 | 1 Body To 14 Bonnet | 51.128 | 88.648 | 368.3 | 0.1388 | 0.2407 | 825 |
| 5 | 1 Body To 14 Bonnet | 54.369 | 42.58 | 368.3 | 0.1476 | 0.1156 | 825 |
| 6 | 1 Body To 14 Bonnet | 268.46 | 69.969 | 368.3 | 0.7289 | 0.19 | 825 |
| 7 | 1 Body To 14 Bonnet | 56.667 | 167.99 | 368.3 | 0.1539 | 0.4561 | 825 |
| 8 | 1 Body To 14 Bonnet | 1210.7 | 176.39 | 368.3 | 3.2873 | 0.4789 | 825 |
| 9 | 1 Body To 14 Bonnet | 1451.3 | 127.84 | 368.3 | 3.9405 | 0.3471 | 825 |
| 10 | 1 Body To 14 Bonnet | 469.77 | 150.03 | 368.3 | 1.2755 | 0.4074 | 825 |
| 11 | 1 Body To 14 Bonnet | 51.052 | 140.56 | 368.3 | 0.1386 | 0.3816 | 825 |
| 12 | 1 Body To 14 Bonnet | 268.62 | 121.9 | 368.3 | 0.7294 | 0.331 | 825 |
| 13 | Outlet Flange To 1 Body | 584.8 | 4764.9 | 303.4 | 1.9275 | 15.705 | 825 |
| 14 | Outlet Flange To 1 Body | 490.16 | 5328.7 | 303.4 | 1.6156 | 17.5633 | 825 |
| 15 | Outlet Flange To 1 Body | 196.48 | 5680.9 | 303.4 | 0.6476 | 18.7241 | 825 |
| 16 | Outlet Flange To 1 Body | 192.72 | 5685 | 303.4 | 0.6352 | 18.7376 | 825 |
| 17 | Outlet Flange To 1 Body | 465.77 | 5327.1 | 303.4 | 1.5352 | 17.558 | 825 |
| 18 | Outlet Flange To 1 Body | 569.14 | 4793.9 | 303.4 | 1.8759 | 15.8006 | 825 |
| 19 | Outlet Flange To 1 Body | 433.05 | 4249.2 | 303.4 | 1.4273 | 14.0053 | 825 |
| 20 | Outlet Flange To 1 Body | 227.85 | 3845.7 | 303.4 | 0.751 | 12.6753 | 825 |
| 21 | Outlet Flange To 1 Body | 61.518 | 3637.6 | 303.4 | 0.2028 | 11.9895 | 825 |
| 22 | Outlet Flange To 1 Body | 211 | 3812.1 | 303.4 | 0.6955 | 12.5646 | 825 |
| 23 | Outlet Flange To 1 Body | 437.47 | 4209 | 303.4 | 1.4419 | 13.8728 | 825 |
| 24 | Outlet Flange To 1 Body | 35.951 | 3623.9 | 303.4 | 0.1185 | 11.9443 | 825 |
| 25 | Inlet Flange To 1 Body | 1875.9 | 2663 | 303.4 | 6.1829 | 8.7772 | 825 |
| 26 | Inlet Flange To 1 Body | 629.98 | 2446.8 | 303.4 | 2.0764 | 8.0646 | 825 |
| 27 | Inlet Flange To 1 Body | 703.56 | 2452.3 | 303.4 | 2.3189 | 8.0827 | 825 |
| 28 | Inlet Flange To 1 Body | 1983.1 | 2690.7 | 303.4 | 6.5363 | 8.8685 | 825 |
| 29 | Inlet Flange To 1 Body | 2550.4 | 3171 | 303.4 | 8.4061 | 10.4515 | 825 |
| 30 | Inlet Flange To 1 Body | 2051.5 | 3795.2 | 303.4 | 6.7617 | 12.5089 | 825 |
| 31 | Inlet Flange To 1 Body | 1453.8 | 4324.8 | 303.4 | 4.7917 | 14.2544 | 825 |
| 32 | Inlet Flange To 1 Body | 541.12 | 4622.6 | 303.4 | 1.7835 | 15.236 | 825 |
| 33 | Inlet Flange To 1 Body | 501.07 | 4615.1 | 303.4 | 1.6515 | 15.2113 | 825 |
| 34 | Inlet Flange To 1 Body | 2618.2 | 3730.8 | 303.4 | 8.6295 | 12.2966 | 825 |
| 35 | Inlet Flange To 1 Body | 2815.9 | 3130.3 | 303.4 | 9.2811 | 10.3174 | 825 |
| 36 | Inlet Flange To 1 Body | 1500.7 | 4293.3 | 303.4 | 4.9463 | 14.1506 | 825 |
| 37 | 14 Bonnet To 25 Top Cover | 0.9539 | 2.3069 | 20.1 | 0.0475 | 0.1148 | 320 |
| 38 | 14 Bonnet To 25 Top Cover | 0.4335 | 1.1854 | 20.1 | 0.0216 | 0.059 | 320 |
| 39 | 14 Bonnet To 25 Top Cover | 17.654 | 0.8494 | 20.1 | 0.8783 | 0.0423 | 320 |
| 40 | 14 Bonnet To 25 Top Cover | 16.868 | 0.8773 | 20.1 | 0.8392 | 0.0436 | 320 |
| 41 | 14 Bonnet To 25 Top Cover | 1.1749 | 2.2736 | 20.1 | 0.0585 | 0.1131 | 320 |
| 42 | 14 Bonnet To 25 Top Cover | 6.0882 | 2.2829 | 20.1 | 0.3029 | 0.1136 | 320 |
| 43 | 14 Bonnet To 25 Top Cover | 14.13 | 2.9336 | 20.1 | 0.703 | 0.146 | 320 |
| 44 | 14 Bonnet To 25 Top Cover | 12.988 | 3.0754 | 20.1 | 0.6462 | 0.153 | 320 |
| 45 | 14 Bonnet To 25 Top Cover | 5.5561 | 2.3065 | 20.1 | 0.2764 | 0.1148 | 320 |
| 46 | 14 Bonnet To 25 Top Cover | 3.9658 | 2.0464 | 20.1 | 0.1973 | 0.1018 | 320 |
| 47 | 14 Bonnet To 25 Top Cover | 4.3522 | 2.0572 | 20.1 | 0.2165 | 0.1023 | 320 |
| 48 | 14 Bonnet To 32 Side Cover | 2.6198 | 2.5141 | 20.1 | 0.1303 | 0.1251 | 320 |
| 49 | 14 Bonnet To 32 Side Cover | 44.291 | 8.3372 | 20.1 | 2.2035 | 0.4148 | 320 |
| 50 | 14 Bonnet To 32 Side Cover | 2.5569 | 2.5721 | 20.1 | 0.1272 | 0.128 | 320 |
| 51 | 14 Bonnet To 32 Side Cover | 43.676 | 7.9931 | 20.1 | 2.1729 | 0.3977 | 320 |

### Shock Analysis Conclusion

Table 36 - Transient Shock Response Summary

| Sr. No. | Material | Static Allowable Stress (MPa) | Shock in Positive X Direction Maximum Occurs on | Shock in Positive X Direction Stresses Observed (MPa) | Shock in Positive Y Direction Maximum Occurs on | Shock in Positive Y Direction Stresses Observed (MPa) | Shock in Positive Z Direction Maximum Occurs on | Shock in Positive Z Direction Stresses Observed (MPa) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ASTM B367 Gr 5 | 256 | 1 Body | 77.317 | 1 Body | 40.128 | 1 Body | 99.793 | Stresses less than Static allowable |
| 2 | ASTM D4066 Nylon66 | 28 | 3 Disc Seal | 6.882 | 3 Disc Seal | 4.511 | 3 Disc Seal | 17.788 | Stresses less than Static allowable |
| 3 | ASTM A217 CA15 | 177 | 28 Hand Wheel | 22.546 | 28 Hand Wheel | 15.948 | 28 Hand Wheel | 42.920 | Stresses less than Static allowable |
| 4 | BS970 EN19 | 199 | 5 Key | 36.572 | 5 Key | 17.590 | 5 Key | 31.116 | Stresses less than Static allowable |
| 5 | ASTM A240 S32100 | 138 | 32 Side Cover | 73.224 | 32 Side Cover | 35.963 | 32 Side Cover | 56.866 | Stresses less than allowable |
| 6 | ASTM B348 Gr 5 | 256 | 11 Stem Cover | 52.234 | 2 Disc | 22.371 | 2 Disc | 113.782 | Stresses less than Static allowable |
| 7 | ASTM A276 S32100 | 138 | 22 Ring Nut | 2.688 | 22 Ring Nut | 2.131 | 22 Ring Nut | 5.142 | Stresses less than Static allowable |
| 8 | BS970 EN36C | 286 | 20 Pinion Shaft | 52.419 | 20 Pinion Shaft | 7.405 | 20 Pinion Shaft | 105.761 | Stresses less than Static allowable |
| 9 | BS2874 CA104 | 186 | 12 Bush-1 | 63.330 | 12 Bush-1 | 18.925 | 12 Bush-1 | 125.832 | Stresses less than Static allowable |

# CFD Analysis

## Introduction

CFD analysis of a valve is required to understand and optimize fluid flow distribution inside it. The study will be helpful in predicting the:

- Pressure distribution in the valve.

- Identifying high velocity regions and vortex shedding.

- Flow distribution leading to losses.

- Sizing of valve using valve coefficient.

- Reduce prototype testing costs

Figure 146(a) shows the computational domain used for the CFD analysis as per EP 1581. The complete valve geometry along with its sectional view is presented in Figure 146(b).

| (a) | (b) |
| --- | --- |

Figure 149 (a) Schematic of valve and its (b) sectional view.

The fluid domain is extracted from the above-mentioned geometry. The inlet and outlet pipe sections are extended beyond 5D, where D represents the pipe bore diameter, to allow the flow to become fully developed prior to entering the valve. The extracted fluid region and the fluid domain is presented in Figure 147.

Bore diameter: 40 mm

Extended fluid domain: 5×50 mm = 200 mm

The extended pipe region of the fluid is 200 mm.

Figure 150 Extracted fluid domain of EP 1581Valve.

## CFD Analysis Methodology

Incompressible, Steady state CFD analysis has been carried out in Valve fully open condition to calculate the flow coefficient of Valve.

## Assumptions

- No slip boundary condition has been considered at the wetted surfaces.

- No mass imbalance between the inlet and outlet is considered.

- No transient effects are considered in the analysis.

## CFD Mesh

In a CFD simulation, the generated grids discretize the fluid domain into small computational volumes. The governing differential equation are solved numerically using these grids.

A properly generated grid helps capture important flow features such as velocity gradients, pressure variations, and boundary layer effects. Mesh quality affects the accuracy, stability, and convergence of solution. Therefore, a proper grid is essential for obtaining reliable CFD results.

In the present simulation, the control volume is discretized using poly hexacore mesh. The polyhedral grids are generated at the surface and inner core is hexahedral. Proper successive increasing grids are generated at the boundaries with three layers of inflation. The aspect ratio and skewness have been kept well within the allowable limits. The aspect ratio is less than 100 and skewness is less than 0.8. The number of cells generated in control volume for simulation is 1044217.

Figure 151 CFD Mesh Size

The grids generated on the external surface and inner core of the fluid domain of the valve is presented in Figure 149.

| (a) |  |
| --- | --- |

Figure 152 Polyhedral grids generated on the exterior surface and hexahedral grids on the internal surface of the valve.

## Material Selection

Sea water flow through the valve EP 1581. The density of Sea water is 1025 kg/m3. Viscosity of Sea water is 0.001003 kg/(m-s). Material properties of Sea water are shown in Figure 150.

Figure 153 Material properties of sea-water.

## Turbulent Viscous model

Turbulence modeling is essential for accurate and feasible simulation of real-world turbulent flows. Turbulence model is required in CFD to account for the effects of turbulent fluctuations on the mean flow without resolving all turbulent scales. Turbulence models provide closure to the Reynolds-averaged Navier–Stokes equations by modeling the additional stress terms.

The mass flow (m*) in the valve is 110 m3/hr.

Bore diameter (D) is 0.04 m.

Area of pipe (A) = π×D24 = 0.001963 m2

Velocity of sea water at the valve inlet = m*ρA×3600   = 15.56 m/s

Reynolds number = Re=ρVDμ = 4.5×105

Reynolds number is more than 4000. Therefore, the flow is turbulent in nature.

The k–ε turbulence model with enhanced wall treatment is used to accurately predict turbulent flows near solid boundaries. Enhanced wall treatment relaxes strict y⁺ requirements but performs best when the mesh is fine enough to achieve y⁺. Therefore, the k–ε model with enhanced wall treatment is well suited for complex internal flows such as those in valves and piping systems.

The k–ε turbulence model with enhanced wall function is selected which is shown in Figure 151.

Figure 154 Selection of turbulence model.

## Boundary Conditions

To simulate sea waterflow through the valve EP1581, the following boundary conditions have been used:

Table 37 Boundary conditions.

| Sl. No. | Boundary | Boundary conditions |  |
| --- | --- | --- | --- |
| 1. | Inlet | Pressure inlet | 4000000 Pa |
| 2. | Outlet | Mass flow outlet | 31.32 kg/s |
| 3. | Wall | Wall | No-slip |

## Solution Methods

The flow in the valve is considered to be steady and incompressible. Pressure velocity coupling has been achieved using Coupled method. Pressure, momentum and turbulence have been discretized using second order upwind scheme.

### Convergence criteria

The flow in the valve is considered to be steady and incompressible. Pressure velocity coupling has been achieved using Coupled method. Pressure, momentum and turbulence has been discretized using second order upwind scheme.

Convergence criteria

The convergence criteria are set at 10-3 for mass momentum and turbulence equations. Other than this it is also checked that that mass balance has been achieved.

Figure 155 Mass Balance.

The residual plots obtained for simulation has been presented in Figure 153 which shows that residuals have reached lower than 10-3. Mass balance has also been achieved. Thus, the solution is converged.

Figure 156 Residual plots.

## Results

### Y+ Plot

The Y+ contours obtained for the flow conditions is plotted in Figure 154. The plot shows distribution of Y+ from 30 to 300. However, wall enhanced treatment has been used for turbulence modelling. Therefore, the Y+ values will not affect the results.

|  | Figure 157 Y+ contours on the computational domain. |
| --- | --- |

### Velocity distribution

The velocity distribution in the valve has been presented in Figure 155. The velocity increases at the orifice of the valve. A high velocity region if 4.26 m/s is observed at the exit of the valve. This indicates that the flow will be incompressible.

Figure 158 Velocity contours in the computational region of EP 1581.

The velocity streamlines are shown in Figure 159 to visualize the eddy dissipation in the valve EP 1581. The lines show that the flow in the valve is nearly smooth until it reaches the orifice region of the valve.

Figure 159 Velocity streamlines in the computational region of EP 1581.

### Pressure distribution

The pressure contour in the valve is presented in Figure 160. The pressure distribution shows that there is a drop-in pressure after orifice region. A maximum pressure of 4000000 Pa is supplied at the inlet. The pressure drops to 3994723.4 Pa at the outlet. A pressure drop of 3734 Pa is obtained in the valve for a mass flow rate of 31.32 kg/s.

Figure 160 Pressure contours in the computational region of EP 1581.

Area averaged pressure obtained at the inlet and outlet of the valve is Pressure at inlet and outlet is 3998457 Pa and 3994723.4 Pa, respectively.

Figure 161 Area averaged pressure obtained at the inlet and outlet

### Calculation of valve coefficient

The valve coefficient (commonly denoted as Cv) indicates how much fluid can flow through a valve for a specified pressure drop when the valve is fully open.

It is defined as the flow rate of water (in US gallons per minute) at 60°F that passes through a valve with a pressure drop of 1 psi.

Table 38 Parameters used for calculation of valve coefficient.

|  | SI Unit |  |  | US Unit |  |
| --- | --- | --- | --- | --- | --- |
| Inlet pressure (Pin) | 3998457 | [Pa] |  | 579.91 | [psi] |
| Outlet Pressure (Pout) | 3994723 | [Pa] |  | 579.38 | [psi] |
| Mass flow rate (m*) | 31.32 | [kg/s] |  |  |  |
| Volume flow rate | 110 | [m3/hr] |  | 484.308 | gpm |
| Pressure difference (∆P) | 3734 | [Pa] |  | 0.54 | [psi] |
| Density of seawater (ρ) | 1025 | [kg/m3] |  |  |  |
| Density of water (ρo) | 1000 | [kg/m3] |  |  |  |

ρρo=1.025

Valve coefficient			 Cv=QN1ρρo∆P	= 708.79

The coefficient of valve obtained from CFD simulation is 708.79 gpm/psi0.5.

### CFD Analysis Conclusions

The Achieved CV value is 708.79 which is greater than the required value of 4, thus the flow path established through design of the is satisfactory as per requirement.

# Material Properties

| Material | UNS | Material Type | Density | Young’s Modulus @ 50 ℃ | Poisson’s ratio | Reference | Yield Strength | Ultimate Tensile Strength | Elongation |  | Static Allowable | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | kg/m3 | GPa |  |  | MPa | MPa | % |  | MPa |  |
| ASTM B 367 Gr.5 | R56400 | Non-Ferrous (Excluding Aluminum) | 4430 | 114.3 | 0.33 | Metals Handbook VOL 2, Properties and Selection: Nonferrous Alloys and Special-Purpose Materials, Introduction to Titanium and Titanium Alloys, 1781 ASME BPVC Sec II ,P-D, 2019, Table PRD , Page no. 899 for Density & Possion ratio and Table TM-5 -Modulus of Elasticity for Titanium, 2019 | 825 | 895 | 6 Refer Note (4) | TABLE S6-1,TENSILE REQUIREMENTS, ASME BPVC.II.B-2023,542. ASTM B367: 2013 Sec II Part-B, Table S6-1 Tensile Requirements, Page no. 552 @ C-5 | 256 | Refer Note (1) |
| ASTM B 348 Gr.5 | R56400 | Non-Ferrous (Excluding Aluminum) | 4430 | 114.3 | 0.33 |  | 825 | 895 | 10 | TABLE 2 Tensile Requirements, ASME BPVC.II.B-2023,504. ASTM B348: 2013 Sec II Part-B, Table 2 Tensile Requirements, Page no. 511 @ Grade-5 | 256 | Refer Note (1) |
| BS 2874 CA 104 | C63000 Refer Note (3) | Aluminum | 7580 | 120 | 0.33 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 320 | 650 | 12 | BS2874, Pg 10 | 186 | Refer Note (1) |
| BS 970 EN19 | G41400 | Ferrous | 7750 | 202.67 | 0.3 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 525 | 695 | 22 | BS 970, Pg 92 (47 of PDF) | 199 | Refer Note (1) |
| BS 970 EN36C | G93100 | Ferrous | 7750 | 189.67 | 0.3 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 850 | 1000 | 13 | BS 970, Pg 128 (65 of PDF) | 286 | Refer Note (1) |
| ASTM A 276 S32100 | S32100 | Ferrous | 8030 | 193 | 0.31 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 205 | 515 | 30 | Table 2 Mechanical Requirements, ASTM276 | 138 | Refer Note (1) |
| ASTM A 217 CA15 | J91150 | Ferrous | 7700 | 199 | 0.31 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 450 | 620 | 18 | Table 1A, ASME BPVC.II.D.M-2023, 116-118 | 177 | Table 1A, ASME BPVC.II.D.M-2023, 52-53-54 |
| ASTM A 240 S32100 | S32100 | Ferrous | 8000 | 193 | 0.31 | ASME BPVC Section II, D, 2023 Pg. 1148 to 1155 | 205 | 515 | 30 | Table 1A, ASME BPVC.II.D.M-2023, 116-118 | 138 | Table 1A, ASME BPVC.II.D.M-2023, 112-114 |
| ASTM D 4066 -- |  | Non-Ferrous (Excluding Aluminium) | 1150 | 3.3 | 0.4 |  | 84 | 85 | 50 |  | 25 | ASTM D 4066 -- |

Notes

- If Not Available in Table 1A the values are calculated based on min (2*YS/3, UTS/3,5). For Bolting Applications, the Yield Strength is considered as the Allowable.

- The Nearest ISO designation is CuAl10Fe5Ni5, the UNS is corresponding to CuAl10Fe5Ni5 is C95820.

- The Nearest ISO designation is CuAl10Ni5Fe4, the UNS is corresponding to CuAl10Ni5Fe4 is C63000.

- As per the information received, the material is suggested by the DMDE.

# Design calculations of the Valve

Pressure Rating of the Valve & Design Validation Valve Wall Thickness Calculations as per ASME B16.34-2020 is not available in case of the Titanium Alloys. Thus, the design of the thickness is validated here with Lames Theory of Thick Cylinders.

|  |  | Stresses at Inner Surface (r = ri) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Input | Internal Body Test Pressure | 8 | MPa | PO Specifications |
| 2 | Input | Minimum Internal Radius (ri) | 75 | mm | Dwg |
| 3 | Input | Minimum thickness | 15 | mm | Dwg |
| 4 | Input | Outside Radius (ro) | 90 | mm | Dwg |
| 5 | OUTPUT | Radial Stress (σr_inner): | -8 | MPa | Lame's equation: -Pi |
| 6 | OUTPUT | Tangential (Hoop) Stress (σt_inner): | 8 * (90^2 + 75^2) / (90^2 - 75^2) = 44.36364 | MPa | Lame's equation: Pi * (ro^2 + ri^2) / (ro^2 - ri^2) |
| 7 | OUTPUT | Axial Stress (σl_inner): | 8 * 75^2 / (90^2 - 75^2) = 18.18182 | MPa | Lame's equation: Pi * ri^2 / (ro^2 - ri^2) |
| 8 | OUTPUT | Von Mises Stress (σv_inner): | sqrt(3) * 8 * 90^2 / (90^2 - 75^2) = 45.34824 | MPa | Lame's simplified equation: sqrt(3) * Pi * ro^2 / (ro^2 - ri^2) |
| 9 | OUTPUT | Material Allowable Stress: | 256 | MPa | ASME BPVC Sec II Part D |
| 11 | OUTPUT | Comparison Result: | ACCEPTABLE |  | Comparison of stresses |
| 12 | OUTPUT | Status: | The calculated Von Mises stress is within the material's allowable limits. |  | Result of comparison |

## Design Validation of the Union Bolted Joint and (Body- Bonnet) Flange

According to Mandatory Appendix 2, Rules for Bolted Flange Connections with Ring Type Gaskets, ASME BPVC.VIII.1-2023, Pg. 337 Onwards.

Figure 115 Union Bolted Joint and (body- bonnet) Flange

### Bolt load calculations

Table 39 Union Bolt Design

| Sl. No. | IN/OP | Parameter | Notation/ Formula | Value | Unit | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Input | Working Pressure (P) | P | 4 | MPa | PO |
| 2 | Input | Bolt Material | ASTM B 348 Gr.5 |  |  |  |
| 3 | Input | Static Allowable for Bolt Material | Sa | 256 | MPa | Table 3, ASME BPVC.II.D.M-2023,426-428, Line 1 |
| 4 | Input | Gasket Factor, m | m | 0 |  | Table 2-5.1, Gasket Materials and Contact Facings, Gasket Factors m for Operating Conditions and Minimum Design Seating Stress y, Pg 345, Section VIII Div 1, ASME BPVC, 2023 |
| 5 | Input | Gasket Factor, y | Y | 0 | MPa |  |
| 6 | Input | Gasket Width | N | 10 | mm | Table 10, Pg. 12 of Gost 12815-80 |
| 7 | Input |  | b0=N/2 = 5/2 | 5 |  | Table 2-5.2, Effective Gasket Width, Pg 347, Section VIII Div 1, ASME BPVC, 2023 |
| 8 | Input | Gasket Seating Width |  | 5 |  |  |
| 9 | Input | Gasket Inside diameter | Di | 210 | mm | Table 10, Pg. 12 of Gost 12815-80 |
| 10 | Input | Number of Bolts |  | 12 |  | Table 10, Pg. 12 of Gost 12815-80 |
| 11 | Input | Diameter of Gasket Load Reaction, The Gasket Load Reaction Point Taken as Gland Outside Diameter for Maximum Preload condition |  | 230 |  | MANDATORY APPENDIX 2, Section2-3, Pg. 338 Section VIII Div 1, ASME BPVC, 2023 |
| 12 | Output | Operating Load |  | Pi/4*230^2*4=16619 0.2514 |  | MANDATORY APPENDIX 2, Section2-5, Pg. 340 Section VIII Div 1, ASME BPVC, 2023 |
| 13 | Output | Gasket seating load |  | 3.14*5*230*0 = 0 |  | MANDATORY APPENDIX 2, Section2-5, Pg. 344 Section VIII Div 1, ASME BPVC, 2023 |
| 14 | Output | Bolt Load | MAX (WM1, WM2) | 166190.2514 |  |  |
| 15 | Output | Total cross‐sectional area of bolts required | Bolt Load/ Static allowable of the bolt material | 166190.2514/256 = 649.1806694 | mm^2 |  |
| 16 | Output | Cross-sectional area per bolt (required) (Am) | Total area required/ Number of Bolts | 649.1806694/12= 54.09838912 | mm^2 |  |
| 17 | Input | Nominal Bolt Size | Dbsc | 24 | mm |  |
| 18 | Input | Pitch | P | 3 | mm |  |
| 19 | Output | Actual Cross-Sectional Area (Ab) | (PI*(Dbsc-0.9382P) ^2)/4 | (PI*(24-0.9382*3) ^2)/4=352.5041494 | mm^2 | Table H-1M Bolt Root and Tensile Stress Areas (Metric Threads), Appendix-H, at Pg. 59 of ASME PCC-1–2019 |
| 20 | Output | FOS | Ab/Am | 352.5041494/352.5041494 = 6.515982364 |  |  |
| 21 | Output | Comparison Result: | ACCEPTABLE |  |  | Comparison of Areas as per Section 2.5 (d) at Pg. 344 of Section VIII Div 1, ASME BPVC, 2023 |
| 22 | Output | Status: | Ab> Am thus, M24×3 is safe for use in this design. |  |  | Result of comparison |
| 23 | Output | Bolt Preload Required | Bolt Load/ Number of Bolts | 166190.2514/12 = 13849.18761 | N |  |
| 25 | Output | Tightening Torque |  | 0.13*24*13849.18761= 43209.46536. | Nmm | ASME PCC 1 |

| Sr. No | IN/OP | Parameter | Value | Unit | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Input | dbsc (Nominal Dia) | 24 | mm |  |
| 2 | Input | P (Pitch) | 3 | mm |  |
| 3 | Input | Bolt UTS | 895 | MPa | TABLE 2 Tensile Requirements, ASME BPVC.II.B-2023,504 |
| 4 | Input | Nut UTS | 895 | MPa | TABLE 2 Tensile Requirements, ASME BPVC.II.B-2023,504 |
| 5 | Input | Tolerance Grade | 6 | ~ |  |
| 6 | Input | pos_ext | g' | ~ |  |
| 7 | Input | pos_int | H' | ~ |  |
| 8 | Input | Target FOS (Input) | 2 | ~ | User Target |
| 9 | Output | THREAD TENSILE STRESS AREA (As) | 352.505 | mm2 | ASME B1.13M Section B-1 |
| 10 | Output | dmin (Min Major Dia - Bolt) | 23.577 | mm | Table 14, ASME B1.13M |
| 11 | Output | d2min (Min Pitch Dia - Bolt) | 21.803 | mm |  |
| 12 | Output | D1max (Max Pitch Dia - Nut) | 21.252 | mm | Table 15, ASME B1.13M |
| 13 | Output | D2max (Max Minor Dia - Nut) | 22.316 | mm |  |
| 14 | Output | NUT THREAD SHEAR AREA (Asn) | 1916.683 | mm2 | ASME B1.13M Section B-2 |
| 15 | Output | BOLT THREAD SHEAR AREA (Ass) | 1410.019 | mm2 |  |
| 16 | Output | Tensile strength of Bolt | 315.492 | kN | ASME B1.13M Section B-3 |
| 17 | Output | Shear strength of Nut Threads | 857.716 | kN |  |
| 18 | Output | Shear strength of Bolt Thread | 630.983 | kN |  |
| 19 | Output | LENGTH OF ENGAGEMENT | 34.847 | mm | Calculated |
| Note: It is recommended to increase the Tap Hole length up to 35 mm. |  |  |  |  |  |

ASn =3.1416pLEd min. p2+0.57735d min. -D2 max. ASs =3.1416pLED1 max. p2+0.57735d2 min. -D1 max. ASn =3.14163(34.817*23.577)32+0.5773523.577 -22.316)  =1916.68 ASs =3.14163(34.817*22.210) 32+0.5773521.803 -21.252) =1410.019

Where, ASn = minimum thread shear area for internal threads, mm2

ASs = minimum thread shear area for external threads, mm2

LE = length of engagement

## Flange moments

| Sl. No. | IN/OP | Parameter | Formula And Unit If Any | Value | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Input | Flange Design Bolt Load (W) | N | 166190.2514 |  |
| 2 | Input | Outside Diameter of Flange (A) | mm | 310 | DWG & Pg. 338 of ASME BPVC Sec VIII, Div 1 |
| 3 | Input | Inside diameter of flange (B) | mm | 210 |  |
| 4 | Input | Bolt‐circle diameter (c) | mm | 265 |  |
| 5 | Input | Thickness of hub at small end, for other integral type flanges (g0) | mm | 20 |  |
| 6 | Input | Thickness of hub at back of flange (g1) | mm | 20 |  |
|  |  | g1/g0 |  | 1 |  |
| 7 | Input | Flange Thickness (t) | mm | 40 |  |
| 8 | Input | Gasket Factor (m) |  | 0 | Table 2-5.1, Pg. 345 of ASME BPVC.VIII.1-2023 |
| 9 | Input | Min. Design Seating Stress (y) |  | 0 |  |
| 10 | Input | G (mm) Dia of Gland seat | mm | 230 | Gland Major Diameter, Location of Load Reaction |
|  | Input | Hub Length (h) | mm | 70 | Pg. 338 of ASME BPVC Sec VIII, Div 1 |
| 11 | Input | Factor, h0=√(B*g0 ) | √(B*g0 ) = √(210*20 ) | 64.80740698 | Pg. 338 of ASME BPVC Sec VIII, Div 1 |
| 12 | Input | Factor, K=(A/B) | A/B =310/210 | 1.476190476 | Figure 2-7.1, of ASME BPVC Sec VIII, Div 1 |
| 13 | Input | Factor Z, involving K |  | 1.472+11.472-1 = 2.696153846 |  |
| 14 | Input | Factor Y, involving K |  | 11.47-10.66845+5.716901.472log101.471.472-1= 5.156519083 |  |
| 15 | Input | Factor U, involving K |  | 1.4721+8.55246 log10*1.47-11.361361.472-1(1.47-1)= 5.666491835 |  |
| 16 | Input | T= factor involving K |  | 1.4721+8.55246log10*⁡1.47-11.04720+1.9448*1.472(1.47-1) = 1.721042321 |  |
| 17 | Input | Factor, V |  | 0.550103 | factor for integral type flanges (from Figure 2-7.3), of ASME BPVC SEC VIII, DIV I |
| 18 | Input | Factor, f |  | 1 | Figure 2-7.6, for g1/g0=1, of ASME BPVC SEC VIII, DIV I |
| 19 | Input | Factor, B1 | B+g0 = 210+20 | 230 | B + go for integral type flanges when f is equal to or greater than one, at Pg. 338 of ASME BPVC SEC VIII DIV I |
| 20 | Input | Factor, F |  | 0.90892 | factor for integral type flanges (from Figure 2-7.2), of ASME BPVC SEC VIII, DIV I |
| 21 | Input | Factor, d |  | 5.6670.550103*64.81*202 = 267026.8241 | Pg. 338 of ASME BPVC SEC VIII DIV I |
| 22 | Input | Factor, e |  | 0.90892/64.81 = 0.01402494 | Pg. 338 of ASME BPVC SEC VIII DIV I |
| 23 | Input | Factor, L |  | 40*0.01402+11.72+403267026.82 = 1.146683395 | Pg. 339 of ASME BPVC SEC VIII DIV I |
| 24 | Input | Factor, R |  | 265-2102-20 = 7.5 | Pg. 339 of ASME BPVC SEC VIII DIV I |
| 25 | Input | Static Allowable for ASTM B 367 Gr.5 | MPa | 256 | Table 1A, ASME BPVC.II.D.M-2023, 116-118 |
| 26 | Output | hd |  | 7.5+0.5*20 = 17.5 | Table 2-6, Moment Arms for Flange Loads Under Operating Conditions, at Pg.348 of ASME BPVC SEC VIII Div I |
| 27 | Output | hg |  | (265-230)/2 = 17.5 |  |
| 28 | Output | ht |  | 17.5+20+17.52 = 22.5 |  |
| 29 | Output | H |  | 0.785*230^2*4 = 166106 | Pg. 338 of ASME BPVC SEC VIII DIV I |
| 30 | Output | HD |  | 0.785*210^2*4= 138474 | Pg. 339 of ASME BPVC SEC VIII DIV I |
| 31 | Output | HG |  | 166190.2514 - 166106 = 84.2513749 | as Wm1-H=0 |
| 32 | Output | HT |  | 166106-138474 = 27632 | Pg. 339 of ASME BPVC SEC VIII DIV I |
| 33 | Output | MT |  | 27632*22.5 = 621720 | Components of the Moment due to HT, HD, HG |
| 34 | Output | MD |  | 138474* 17.5 = 2423295 |  |
| 35 | Output | MG |  | 84.251*17.5 = 1474.399061 |  |
| 36 | Output | total moment acting upon the flange, for the operating conditions | MT+MD+MG | 621720 + 2423295 + 1474.399061 = 3046489.399 | M0=MG+MT+MD |
| 37 | Output | Longitudinal hub stress (SH) |  | 1*3046489.3991.1467*202*210 = 31.62837371 | 2-7 CALCULATION OF FLANGE STRESSES, at Pg. 346 of ASME BPVC SEC VIII, Div I |
| 38 | Output | Radial flange stress (SR) |  | 1.33*40*0.01402+1*3046489.3991.1467*402*210 = 13.80678784 |  |
| 39 | Output | Tangential flange stress (ST) |  | 5.156*3046489.399402*210-2.696*13.8067 = 9.528587526 | 2-7 CALCULATION OF FLANGE STRESSES, at Pg. 348 of ASME BPVC SEC VIII, Div I |
|  |  | ALLOWABLE FLANGE DESIGN STRESSES |  |  |  |
| 40 | Output | SH<1.5*Static Allowable |  | 384 | Section 2-8 ALLOWABLE FLANGE DESIGN STRESSES, Para (a) (1) (-b) at Pg. 355 of ASME BPVC SEC VIII Div I |
| 41 | Output | SR<Static Allowable |  | 256 | Section 2-8 ALLOWABLE FLANGE DESIGN STRESSES, Para (a) (2) at Pg. 355 of ASME BPVC SEC VIII Div I |
| 42 | Output | ST<Static Allowable |  | 256 | Section 2-8 ALLOWABLE FLANGE DESIGN STRESSES, Para (a) (3) at Pg. 355 of ASME BPVC SEC VIII Div I |
| 43 | Output | (SH+SR)/2<Static Allowable |  | 256 | Section 2-8 ALLOWABLE FLANGE DESIGN STRESSES, Para (a) (4) at Pg. 355 of ASME BPVC SEC VIII Div I |
| 44 | Output | (SH+ST)/2<Static Allowable |  | 256 | Section 2-8 ALLOWABLE FLANGE DESIGN STRESSES, Para (a) (4) at Pg. 355 of ASME BPVC SEC VIII Div I |
| 45 | Output | Longitudinal hub stress (SH) |  | ACCEPTED | 31.628<384 |
| 46 | Output | Radial flange stress (SR) |  | ACCEPTED | 13.807<256 |
| 47 | Output | Tangential flange stress (ST) |  | ACCEPTED | 9.529<256 |
| 48 | Output | SH+SR |  | ACCEPTED | 45.435<256 |
| 49 | Output | SH+ST |  | ACCEPTED | 41.157<256 |
| 50 | Output | Flange Rigidity |  | 52.14*0.55010*3046489.3991.1467*20o2*0.3*64.807*200*1000= 0.05076059 | Table 2-14 Flange Rigidity Factors at Pg. 358 of ASME BPVC SEC VIII Div I |

## Effort Required

The methodology described in the book entitled “The Design of Valves and Fittings by G.H. Pearson” is used here to calculate the effort required to calculate the effort required to manually operated valve.

Initially, the total force required to operate the valve is calculated as follows

- Total axial force acting on the spindle (Page 36-37)

  - Fa is the total axial load due to Axial force to resist the pressure load (P), Axial Clamping force at seating, and Friction force

Fa=FPressure+Fs+Ff

  - Axial force to resist the pressure load

FPressure=P×π4×d02

Figure 116 components the Fa and Fs

  - Axial clamping force at seating

Fs=2×π×d0×w×P

  - Friction force at Oring Interface (Derived from first Principles)

Ff=μoring×π×P×doring×horing

    - horing is the Height of Oring interfacing with spindle

    - doring is the internal diameter of the Oring

The following image describes the terminology associate with the power screws. (Chapter 4, Page 52)

Figure 117 Single start Right-Handed Square Thread

- The Mean Thread Diameter can be denoted by dt, Dm, D2,

- The p denotes the Pitch of the thread,

- Alpha (α) is thread inclination angle, given by following formula,

tanα=pπ*dt

- Phi (∅) is the friction angle

tan∅=μ

- The friction angle needs the correction as per the thread angle

Table 40 Relation between thread angle and coefficient of frictions

|  | Trapezoidal Thread | Metric Thread | Square Thread |
| --- | --- | --- | --- |
| Angle (β) | 15° | 30° | 0° |
| Adjusted friction μ' | μ'=μcos15 | μ'=μcos30 | μ'=μcos0 |
| Corrected Friction angle (∅') | tan-1μcos15 | tan-1μcos30 | tan-1μ |

- The efficiency of the screw is given by, (Page 58)

ηscrew=tanαtanα+∅'

- The torque required to actuate the spindle is given by, (Page 66)

T=FEffort×RHW=Fa×pitch2×π×ηscrew

FEffort=TRHW

- Length of the Thread Engagement based to bearing pressure.

lt=2Faπdmfp  = 2*123178.7064π*33*99 = 24.001

fp is the maximum allowable intensity of bearing pressure.  The allowable  fp is estimated from the materials compressive / contact strength. Here stem material is NAB BS 2874 CA104. The formula from the below. σy is the yield tensile strength.

#### fp=(0.12 to 0.22)×σy = 0.12*825 = 99

- Length of the Thread Engagement if given by the following formula

Le=2×Faπ×dt×τall

Table 41 Effort required and the length of thread engagement for Spindle

| Sr. No | IN/OP | Parameter | Value | Unit | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Input | Working Pressure, P | 4 | MPa | PO |
| 2 | Input | Disc Dia , do | 177 | mm | DWG |
| 3 | Input | Seating Width (mm), w | 5 | mm | DWG |
| 4 | Input | Handwheel Dia (mm), 2*RHW | 280 | mm | DWG |
| 5 | Input | Mean Diameter of the Thread (Dbsc) | 44 | mm | DWG |
| 6 | Input | Pitch (p) | 7 | mm | DWG |
| 7 | Input | O-ring Dia, doring | 40 | mm | DWG |
| 8 | Input | O-ring width, horing | 20 | mm | DWG |
| 9 | Input | mu O-ring | 0.25 |  | DWG |
|  | Input | Type of Thread | Trapezoidal |  | DWG |
| 10 | Input | Static allowable of Internal thread, Stem Bush | 186 | MPa | Refer to Materials data table |
| 11 | Input | Static Allowable of external thread, Spindle | 256 | MPa | Refer to Materials data table |
| 12 | Input | Allowable Shear stress of Internal thread | 93 | MPa | 0.5 * Static Allowable |
| 13 | Input | Allowable Shear stress of External thread | 128 | MPa | 0.5 * Static Allowable |
| 14 | Output | Force acting on the Disc, Fpressure | 98422.95624 | N | DWG |
| 15 | Output | Axial Clamping force at seating, Fs | 22242.47599 | N | DWG |
| 16 | Output | Friction Force. Ff | 2513.274123 | N | DWG |
| 17 | Output | Total Force, Fa | 123178.7064 | N | DWG |
| 18 | Output | Thread Mean Dia, Dm | 40.5 | mm | DWG |
| 19 | Output | Tan (Alpha) =pπ*dt | 0.055016524 |  | DWG |
| 20 | Output | Alpha | 3.149039972 | Degree | DWG |
| 21 | Output | Mu at metal interface | 0.15 |  | DWG |
| 22 | Output | Thread Angle (Beta) | 15 | Degree | DWG |
| 23 | Output | 1/Cos (Beta) | 1.03527618 |  | DWG |
| 24 | Output | Adjusted mu= Mu at metal interface*1/Cos (Beta) | 0.155291427 |  | DWG |
| 25 | Output | Friction Angle (ATAN(Adjusted mu)), ∅' | 8.82703806 | Degree | DWG |
| 26 | Output | tanα+∅' | 0.21212022 |  | DWG |
| 27 | Output | Screw Efficiency, ηscrew | 0.259364824 |  | DWG |
| 28 | Output | Torque Required, T | 529106.0587 | N mm | DWG |
| 29 | Output | Manual Effort Required | 3779.328991 | N | DWG |
| 30 | Output | Length of Thread Engagement= 2*Fa/pi () *Dm*Sall = 2*123178.7/pi () *40.5 *128 | 15.12692901 | mm | Actual Engagement length is 94 mm |
| 31 | Output | Length of Thread Engagement for bearing pressure (refer sec 103, point ix) | 24.001 | mm | Actual Engagement length is 94 mm |
| 31 | Input | Core Diameter of External Thread, d3 | 36.5 | mm |  |
| 32 | Input | Core Diameter at Internal Thread | 44.5 | mm |  |
| 33 | Output | Thread Thickness at Root of External Thread | 4.58 | mm | Pitch-e (refer figure 118) |
| 34 | Output | Thread Thickness at Root of Internal Thread | 4.58 | mm | Pitch-e (refer figure 118) |
| 35 | Output | Number of Threads in Engagement | 2.160989859 |  | Length of Thread Engagement/ Pitch |
| 36 | Output | Transvers Shear Stress of External Thread | 108.5362206 | MPa | <128 MPa Allowable shear Strength |
| 37 | Output | Transvers Shear Stress of Internal Thread | 89.02409107 | MPa | <93 MPa Allowable Shear Strength |
| Thread Strength in Case of Shock Load |  |  |  |  |  |  |
| 37 | Input | Valve Mass | 110 | Kg |  |
| 38 | Input | Shock Acceleration | 300 | G |  |
| 39 | Input | Inertia Force (Mass * Acceleration) , Shock Load = 9.81*(110*300) | 323730 | N |  |
| 40 | Output | Length of Thread Engagement= 2*Fa/pi ()*Dm*Sall = 2*323730/ pi ()*40.5*128 | 39.75557849 | mm | Actual Engagement length is 94 mm |
|  |  | Length of Thread Engagement with bearing pressure = 2*Fa/pi () *Dm*fp = 2*323730/pi () *40.5 *99 | 51.39 | mm | Actual Engagement length is 94 mm |
| 41 | Output | Core Diameter of External Thread, d3 | 36.5 | mm |  |
| 42 |  | Core Diameter at Internal Thread | 44.5 | mm |  |
| 43 | Output | Thread Thickness at Root of External Thread | 4.58 | mm |  |
| 44 | Output | Thread Thickness at Root of Internal Thread | 4.58 | mm |  |
| 45 | Output | Number of Threads In Engagement | 5.679368356 |  | Length of Thread Engagement/ Pitch |
| 46 | Output | Transvers Shear Stress of External Thread = Force/(Pi*Core Diameter of External Thread*Thread Thickness at Root*Number of Threads in Engagement) | 108.5362206 | MPa | <128 MPa Allowable shear Strength |
| 47 | Output | Transvers Shear Stress of Internal Thread = Force/ (Pi*Core Diameter of Internal Thread*Thread Thickness at Root*Number of Threads in Engagement) | 89.02409107 | MPa | <93 MPa Allowable Shear Strength |

Figure 118 PSG Design Data Book Details of Trapezoidal Threads

## End Connection Flange Design

Figure 119 Typical End Connection Design

Table 42 End Connection Flange Design

| Sr. No | Parameter | Value | Reference |
| --- | --- | --- | --- |
|  | design pressure | 4 MPa | PO |
|  | Nominal Pipe Size (NPS)/ (Dy as per Gost 12815-80) | 150 mm | Table 4.2 |
|  | Outside dia of the end Flange (D) | 280 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | PCD (D1) | 236 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | D2 | 204 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | D3 | 182 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | Depth of Groove (h) | 4 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | Bolt Hole Diameter (d) | 22 mm | Table 10, Pg. 12 of Gost 12815-80 |
|  | Number of Bolts (n) | 12 Nos. | Table 10, Pg. 12 of Gost 12815-80 |
|  | Nominal Diameter of Stud | M20 | Table 10, Pg. 12 of Gost 12815-80 |
|  | Nut Factor, k | 0.13 |  |

Table 43 Tightening torque and preload calculations

| Sl. No. | IN/OP | Parameter | Notation/ Formula | Value | Unit | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Input | Working Pressure (P) | P | 4 | MPa | PO |
| 2 | Input | Bolt Material | ASTM B 348 Gr.5 |  |  |  |
| 3 | Input | Static Allowable for Bolt Material | Sa | 256 | MPa | Table 3,ASME BPVC.II.D.M-2023,422-425, Line 44 |
| 4 | Input | Gasket Factor, m | m | 2 |  | Table 2-5.1, Gasket Materials and Contact Facings,Gasket Factors m for Operating Conditions and Minimum Design Seating Stress y, Pg 345,Section VIII Div 1, ASME BPVC, 2023 |
| 5 | Input | Gasket Factor, y | Y | 11 | MPa |  |
| 6 | Input | Gasket Width | N | 11 |  | Table 10, Pg. 12 of Gost 12815-80 |
| 7 | Input |  | b0=N/2 | 5.5 |  | Table 2-5.2, Effective Gasket Width, Pg 347, Section VIII Div 1, ASME BPVC, 2023 |
| 8 | Input | Gasket Seating Width |  | 5.5 |  |  |
| 9 | Input | Gasket Inside diameter | Di | 182 | mm | Table 10, Pg. 12 of Gost 12815-80 |
| 10 | Input | Number of Bolts |  | 12 |  | Table 10, Pg. 12 of Gost 12815-80 |
| 11 | Input | Diameter of Gasket Load Reaction, The Gasket Load Reaction Point Taken as Gland Outside Diameter for Maximum Preload condition |  | 193 |  | MANDATORY APPENDIX 2, Section2-3, Pg. 338 Section VIII Div 1, ASME BPVC, 2023 |
| 12 | Output | Operating Load |  | 0.785*193^2*4 = 117297.6449 |  | MANDATORY APPENDIX 2, Section2-5, Pg. 340 Section VIII Div 1, ASME BPVC, 2023 |
| 13 | Output | Gasket seating load |  | = 36682.80662 |  | MANDATORY APPENDIX 2, Section2-5, Pg. 344 Section VIII Div 1, ASME BPVC, 2023 |
| 14 | Output | Bolt Load | MAX (WM1, WM2) | 117297.6449 |  |  |
| 15 | Output | Total cross‐sectional area of bolts required | Bolt Load/ Static allowable of the bolt material | 458.1939254 | mm^2 |  |
| 16 | Output | Cross-sectional area per bolt (required) (Am) | Total area required/ Number of Bolts | 38.18282712 | mm^2 |  |
| 17 | Input | Nominal Bolt Size | Dbsc | 20 | mm |  |
| 18 | Input | Pitch | P | 2.5 | mm |  |
| 19 | Output | Actual Cross-Sectional Area (Ab) | (PI*(Dbsc-0.9382P) ^2)/4 | 244.7945482 | mm^2 | Table H-1M Bolt Root and Tensile Stress Areas (Metric Threads), Appendix-H, at Pg. 59 of ASME PCC-1–2019 |
| 20 | Output | FOS | Ab/Am | 6.411116375 |  |  |
| 21 | Output | Comparison Result: | ACCEPTABLE |  |  | Comparison of Areas as per Section 2.5 (d) at Pg. 344 of Section VIII Div 1, ASME BPVC, 2023 |
| 22 | Output | Status: | Ab> Am thus, M20×2.5 is safe for use in this design. |  |  | Result of comparison |
| 23 | Output | Bolt Preload Required | Bolt Load/ Number of Bolts | 9774.803742 | N |  |
| 25 | Output | Tightening Torque, with k=0.13 |  | 25414.48973 | N mm | ASME PCC 1 |